# RoadBounce: Sensor Network for Road Surface Monitoring

Farhan Ali – BSE223048

Tooba Fatima – BSE223124

Laiba Taj – BSE223213

Supervised By

Mam Anum Naseem

FYP Fall 2025

BS Software Engineering Department of Software Engineering Capital University of Science and Technology, Islamabad

|  | Project Report |  |
| --- | --- | --- |

| VERSION |  | V X.0 |  |  | NUMBER OF MEMBERS | 3 |
| --- | --- | --- | --- | --- | --- | --- |
| TITLE | RoadBounce: Sensor Network for Road Surface Monitoring |  |  |  |  |  |
| SUPERVISOR NAME |  |  | Mam Anum Naseem |  |  |  |

| MEMBER NAME | REG. NO. | EMAIL ADDRESS |
| --- | --- | --- |
| Farhan Ali | BSE223048 | fa5973722@gmail.com |
| Tooba Fatima | BSE223124 | fatimatooba0804@gmail.com |
| Laiba Taj | BSE223213 | laibataj6@gmail.com |

| MEMBERS’ SIGNATURES |  |  |
| --- | --- | --- |
|  |  | Supervisor’s Signature |

# Approval Certificate

# This project, entitled as “RoadBounce: Sensor Network for Road Surface Monitoring” has been approved for the award of

Bachelor of Science in Software Engineering

Committee Signatures:

Supervisor:	________________

(Supervisor Name Dr / Mr / Ms)

Project Coordinator:	__________________________

(Mr. Ibrar Arshad)

Head of Department:	__________________________

(Dr. Nadeem Anjum)

# Declaration

We, hereby, declare that “No portion of the work referred to, in this project has been submitted in support of an application for another degree or qualification of this or any other university/institute or other institution of learning”. It is further declared that this undergraduate project, neither as a whole nor as a part thereof has been copied out from any sources, wherever references have been provided.

MEMBERS’ SIGNATURES

# Acknowledgements

We would like to thank our supervisor Ma'am Anum Naseem for her valuable guidance and continuous support and dedicated supervision in the entire course of this project. Her expertise and encouragement had a very strong influence on the direction of our work and on ensuring that we were on track throughout.

Finally, we would like to express our sincere appreciation to Mr. Raheel Bari, Project Director at the Embedded Systems Group at Teresol for his technical knowledge and mentorship. His support and direction were very helpful to our understanding and helped to make this project successful.

# Executive Summary

Poor condition of the roads is a continuous public safety and infrastructure issue in Pakistan that causes thousands of accidents, damages to vehicles and high fuel consumption every year. Current road monitoring techniques are extremely costly, relying on specialist laser profiling vehicles, or slow and unreliability, based on manual inspections, leaving road defects undetected until they happen, when it is already too late.

To fill this need, the RoadBounce system is a real-time road surface monitoring system at a low cost. The system involves sensor hardware that is connected to a microprocessor to continually scan the roadway for anomalies and irregularities. The sensor data is coupled with GPS coordinates and sent to the central server for processing, storing and analysing with machine learning classification.

A mobile application to go with it enables the end-user to have immediate access to map of roads in the area, real time alerts when any local anomalies are detected, and intelligent route suggestions according to the current state of the roads. This data can be used by transportation agencies and authorities to transition from a reactive to proactive approach to road maintenance. This project involves the design and implementation of hardware, embedded systems solutions, the backend data processing, and the development of mobile applications, presenting a comprehensive and scalable end-to-end solution for smart road infrastructure monitoring.

Table of Contents

Approval Certificate	iii

Declaration	iv

Acknowledgements	v

Executive Summary	vi

Glossary	xvi

Chapter 1	1

1	Introduction	1

1.1	Project Introduction	2

1.2	Existing Examples / Solutions	2

1.2.1	Summary of Shortcomings	4

1.2.2	Relevance to Our Project	4

1.3	Business Scope	5

1.3.1	Primary Target Market	5

1.3.2	Secondary Target Market	6

1.3.3	Additional Market Potential	6

1.3.4	Resources Needed for Scaling	6

1.4	Useful Tools and Technologies	9

1.4.1	Useful Hardware	9

1.4.2	Useful Software	14

1.5	Project Work Break Down	16

1.6	Project Timeline	18

Chapter 2	19

2	Requirement Specification and Analysis	19

2.1	Functional Requirements	19

2.2	Non-Functional Requirements	23

2.3	Epics	26

2.3.1	Epic 1	27

2.3.2	Epic 2	28

2.3.3	Epic 3	31

2.3.4	Epic 4	35

2.3.5	Epic 5	36

2.4	Test-cases	37

2.4.1	Test Case 1: Continuous Sensor Data Collection	37

2.4.2	Test Case 2: Sensor Calibration	38

2.4.3	Test Case 3: GPS Integration with Sensor Data	39

2.4.4	Test Case 4: Bandpass Filter for Sensor Data	40

2.4.5	Test Case 5: Classify Road Anomalies with Machine Learning	40

2.4.6	Test Case 6: Real-time Road Anomaly Detection	41

2.4.7	Test Case 7: Clustering of Repeated Anomaly Detections	42

2.4.8	Test Case 8: False Alert Filtering for Environmental Conditions	43

2.4.9	Test Case 9: Road Anomaly Recording and Storage with GPS Coordinates	43

2.4.10	Test Case 10: Marking of Road Anomalies with GPS Coordinate	44

2.4.11	Test Case 11: Prevent data loss/duplication in multi-vehicle streams	44

2.4.12	Test Case 12: Secure Login for Mobile App	45

2.4.13	Test Case 13: Display Road Conditions on Map	46

2.4.14	Test Case 14: Real-Time Alerts for Users on Road Anomalies	47

2.4.15	Test Case 15: Best Route Options Based on Real-Time Data	47

2.4.16	Test Case 16: Intuitive Mobile App Interface	48

2.4.17	Test Case 17: List of History of Trips	49

2.4.18	Test Case 18: Display Anomaly Info Card	49

2.4.19	Test Case 19: Secure and Encrypted Communication	50

2.4.20	Test Case 20: Real-time Data Synchronization	51

2.4.21	Test Case 21: Verify Data Reception and Standardized Storage	51

2.4.22	Test Case 22: Test Sensor Accuracy on Different Roads	52

2.5	User Interface Design (Prototypes)	53

2.5.1	Onboarding Screen	53

2.5.2	Sign-Up Page	54

2.5.3	Sign-In Page	55

2.5.4	User Dashboard (Map Display)	56

2.5.5	Anomaly Info Pop-up Page	57

2.5.6	Route Search page	58

2.5.7	Route Options Selection Page	59

2.5.8	Selected Route Map Display Page	60

2.5.9	User Profile Page	61

2.5.10	User History Page	62

2.5.11	Trip Summary Page	63

2.5.12	Settings Drawer Page:	64

2.6	Traceability Matrix	65

Chapter 3	69

3	System Design	69

3.1	Software architecture	69

3.1.1	Client Server	70

3.1.2	Pipe-Filter	70

3.2	Components and Connectors	71

3.2.1	Sensors and Devices	71

3.2.2	Connectivity	73

3.2.3	Data Processing	73

3.2.4	User Interface	73

3.3	Hardware Specifications	74

3.4	Communication Protocols	78

3.5	Data Modeling	79

3.5.1	NoSQL Database	80

3.6	Workflow Diagram	83

3.7	Third-Parties Dependencies	84

3.7.1	Hardware Dependencies	84

3.7.2	Software Frameworks & Tools	84

3.7.3	External Services & Cloud Infrastructure	85

Chapter 4	86

4	Software Development	86

4.1	Implementation Overview	86

4.2	VSU Software Implementation (Python & Edge Processing)	86

4.2.1	Sensor Interface Layer	86

4.2.2	Signal Processing and Anomaly Detection	87

4.3	Mobile Application Implementation	87

4.3.1	User Interface and State Management	88

4.3.2	Real-Time Map Visualization	88

4.4	Problems Encountered and Solutions	88

4.4.1	Problem 1: Real-Time Data Collection from MPU6050 and GPS	89

4.4.2	Problem 2: Signal Noise and Vehicle Tilt	89

4.4.3	Problem 3: False Positives in Anomaly Detection	90

4.4.4	Problem 4: Repeated Detections for the Same Pothole	91

4.4.5	Problem 5: Low-Speed Noise and False Positives	92

4.4.6	Problem 6: Severity and Impact Interpretation	93

4.5	Coding Standards	94

4.5.1	Indentation	94

4.5.2	Declaration	95

4.5.3	Naming Convention	95

4.5.4	Statement Standard	96

4.6	Development Environment	97

4.6.1	Programming Language Selection	98

4.6.2	Development Platform	98

4.6.3	Data Processing and Scientific Libraries	99

4.6.4	Machine Learning and Deep Learning Frameworks	99

4.6.5	Signal Processing Tools	100

4.6.6	Mobile Application Development Environment	100

4.6.7	Hardware Development Environment	101

4.7	Software Description	102

4.7.1	Sign-In Process	103

4.7.2	User Dashboard	104

4.7.3	Route Selection	105

4.7.4	User History	107

4.7.5	Anomaly Info Card	108

4.7.6	Data Filtering	109

4.7.7	Anomaly Detection	110

4.7.8	Anomaly Recording with GPS Coordinates	112

4.7.9	Environmental Adaptation	113

4.7.10	Anomaly Marking	114

Chapter 5	116

5	Software Deployment	116

5.1	Deployment Process Description	116

5.1.1	Raspberry Pi Deployment	117

5.2	System Requirements for Application	119

5.2.1	Google Play Console Deployment	119

5.3	Deployment Challenges	126

References	128

List of Figures

Figure 1-1: Accelerometer Sensor	10

Figure 1-2: Gyroscope Sensor	10

Figure 1-3: MPU-6050	11

Figure 1-4: NEO-M8N GPS module	11

Figure 1-5: Raspberry-Pi 4 SBC	12

Figure 1-6: 12v/ 5 amp battery	13

Figure 1-7: Step Down Voltage Regulator	13

Figure 1-8: Android Studio Icon	14

Figure 1-9: Google Colab Icon	15

Figure 1-10: Firebase Icon	15

Figure 1-11: Python Icon	15

Figure 1-12: Kotlin Icon	16

Figure 1-13: XML Icon	16

Figure 1-14: Work-Breakdown Structure	17

Figure 1-15: Project Timeline Gantt Chart.	18

Figure 2-1: Onboarding Screen	54

Figure 2-2: Sign-up Page	55

Figure 2-3: Sign-in Page	56

Figure 2-4: User Dashboard (Map Display)	57

Figure 2-5: Anomaly Info Pop-up Page	58

Figure 2-6: Route Search Page	59

Figure 2-7: Route Options Selections Page	60

Figure 2-8: Selected Route Map Display Page	61

Figure 2-9: User Profile Page	62

Figure 2-10: User History Page	63

Figure 2-11: Trip Summary Page	64

Figure 2-12: Settings Drawer Page	65

Figure 3-1: User Collection	81

Figure 3-2: VSU Collection	81

Figure 3-3: Road_Anomaly Collection	82

Figure 3-4: Map_Clusters Collection	83

Figure 3-5: Workflow Diagram	83

Figure 4-1: Combining MPU6050 and GPS Data	89

Figure 4-2: Tilt Correction and Filtering	90

Figure 4-3: CNN Input Preparation and Prediction	91

Figure 4-4: Cooldown Logic	92

Figure 4-5: Speed Threshold Check	92

Figure 4-6: Severity and Impact Calculation	93

Figure 4-7: Anomaly Details	94

Figure 4-8: Class Diagram	103

Figure 4-9: Sign-In Code	103

Figure 4-10: Dashboard Code	104

Figure 4-11: Route Selection Code	106

Figure 4-12: User History Code	107

Figure 4-13: Anomaly Info Card Code	108

Figure 4-14: Filtered Data Consumption Code	109

Figure 4-15: Anomaly Detection Data Listener Code	111

Figure 4-16: GPS Coordinate Parsing Code	112

Figure 4-17: Severity-Based Route Penalty Code	113

Figure 4-18: Anomaly Marker Code	114

Figure 5-1: Google Play Console Dashboard	120

Figure 5-2: Create Application Page	121

Figure 5-3: Uploading Android App Bundle	121

Figure 5-4: Privacy Policy Publication Using Google Sites	122

Figure 5-5: Privacy Policy Configuration	123

Figure 5-6: Data Safety Form Configuration	123

Figure 5-7: Store Listing Configuration	124

Figure 5-8: Publishing Overview and Release Submission	124

Figure 5-9: Closed Testing Release Status in Google Play Console	125

Figure 5-10: Publishing Overview for Release Submission	126

Figure 5-11: Application Under Review in Closed Testing Track	126

List of Tables

Table 1-1: Lean Canvas for RoadBounce	7

Table 2-1: Functional Requirements for RoadBounce	19

Table 2-2: Non-Functional Requirements for RoadBounce	23

Table 2-3: Epic 1 User Stories	27

Table 2-4: Epic 2 User Stories	29

Table 2-5: Epic 3 User Stories	32

Table 2-6: Epic 4 User Stories	35

Table 2-7: Epic 5 User Stories	37

Table 2-8: Continuous Sensor Data Collection	38

Table 2-9: Sensor Calibration	38

Table 2-10: GPS Integration with Sensor Data	39

Table 2-11: Bandpass Filter for Sensor Data	40

Table 2-12: Classify Road Anomalies with Machine Learning	41

Table 2-13: Real-time Road Anomaly Detection	41

Table 2-14: Clustering of Repeated Anomaly Detections	42

Table 2-15: False Alert Filtering for Environmental Conditions	43

Table 2-16: Road Anomaly Recording and Storage with GPS Coordinates	43

Table 2-17: Marking of Road Anomalies with GPS Coordinates	44

Table 2-18: Prevent data loss/duplication in multi-vehicle streams	45

Table 2-19: Secure Login for Mobile App	45

Table 2-20: Display Road Conditions on Map	46

Table 2-21: Real-Time Alerts for Users on Road Anomalies	47

Table 2-22: Best Route Options Based on Real-Time Data	47

Table 2-23: Intuitive Mobile App Interface	48

Table 2-24: List of History of Trips	49

Table 2-25: Display Anomaly Info Card	49

Table 2-26: Secure and Encrypted Communication	50

Table 2-27: Real-time Data Synchronization	51

Table 2-28: Verify Data Reception and Standardized Storage	52

Table 2-29: Test Sensor Accuracy on Different Roads	52

Table 2-30: Traceability Matrix	65

Table 3-1: Hardware Specifications	74

Table 4-1: Naming Convention Table	96

Table 4-2: Third-party python libraries	99

Table 4-3: Secure Login for Mobile App	104

Table 4-4: Display Road Conditions on Map	105

Table 4-5: Best Route Options Based on Real-Time Data	106

Table 4-6: List of History of Trips	108

Table 4-7: Anomaly Info Card	109

Table 4-8: Data Filtering Accuracy	110

Table 4-9: Real-Time Road Anomaly Detection	111

Table 4-10: Road Anomaly Recording and Storage with GPS Coordinates	112

Table 4-11: False Alert Filtering for Environmental Conditions	113

Table 4-12: Marking of Road Anomalies with GPS Coordinates	115

Table 5-1: Files transferred to Raspberry Pi	117

Table 5-2: System Requirements	119

# Glossary

| Term | Definition |
| --- | --- |
| Anomaly | An anomaly is any irregularity in sensor data (in the case of acceleration mostly), which is not aligned with typical road vibrations and can potentially suggest the existence of a pothole or bump. |
| CNN, 1D (One-Dimensional Convolutional Neural Network) | A machine learning model that automatically finds patterns in sequences of numbers over time, like road vibration data. |
| Low level serial communication protocols | Protocols that use a single channel to transmit data bit-by-bit that concentrate on physical signaling and simple framing. |
| Vehicle Sensor Unit (VSU) | The collective hardware composed of Raspberry Pi, accelerator, gyroscope, and GPS device, mounted on a car to record and calculate data in real-time. |
| WebSockets | Communications protocol that allows one client and one server to connect the two endpoints in a persistent, full duplex, two-way TCP connection. |

# Chapter 1

# Introduction

With urban areas getting more technologically advanced and starting to install the smart infrastructure, one of the aspects of the smart infrastructure that is slightly more neglected is the condition of our roads that we use daily. Highways form an important part of social and economic life and with time they wear out. This happens frequently, which results in vehicle damage, consuming more fuel, and/or accident threats.

Traffic safety reports to the Government of Pakistan have cited thousands of accidents and injuries in the last ten years due to cracks, potholes, and missing gutter covers on roadways. The road conditions not only damages vehicles, but it can also put the safety of commuters at risk, making monitoring road safety a prevailing challenge.

According to traffic safety reports to the Government of Pakistan, thousands of accidents and injuries took place in the past ten years on the roadways due to cracks, potholes as well as missing gutter covers. The road behaviour does not only destroy cars, but it may also jeopardize the wellbeing of commuters, thus making it necessary to monitor road safety. Nowadays, surveillance of the roads is costly due to the necessity of specific technology or slow because the process depends on the human checks and manual reviews. This may lead to delays and unreliability of information, which complicate the maintenance of roads by the authorities.

The proposed solution can be viewed as an innovation of a more effective way of checking the level of road maintenance, which involves the application of real-time sensors, which are linked to a central server to process. In case scaling the proposal is possible, it is basically surveillance of roads 24/7 and this will assist the authorities to switch to proactive model of maintenance and prediction of roads rather than reactive model of maintenance. [1].

## Project Introduction

The goal of the project is to develop a budget friendly real-time road monitoring system with the use of sensors and an integrated microprocessor. The road conditions will be monitored using this technology to detect anomalies. Along with the sensor-based monitoring system, the project will also provide a mobile application that will allow users to access real-time data, view road condition maps, and receive alerts about any road issues.

This project's focus is on the significant informational gap that the different authorities and transportation agencies face. The existing methods for the assessment of road quality are either too expensive, requiring specialized laser profiling vehicles, or are slow and inefficient as they rely on manual inspections. It creates a delay in the identification of defects, and that too, some damage or safety risk has been caused. The system will be delivering consistent data which will allow the organizations and authorities to adopt a proactive maintenance strategy.

The project is focused on transforming urban transportation and infrastructure management. It is designed to maintain a very broad spectrum of stakeholders such as the government officials in charge of road maintenance, fleet managers, rideshare and delivery drivers that drive daily, civil engineers, autonomous vehicles that can communicate with one another, and logistic professionals who seek smarter means of transportation.

By providing a scalable, sensor-based system that operates in real-time, the project allows all users to take proactive steps in the maintenance of the roads, which improve efficiency and makes urban mobility safer and intelligent [2].

## Existing Examples / Solutions

Researchers have proposed several approaches for monitoring road damage. Three significant solutions are as follows:

- ShakeSensing: A Mobile Application for the Crowdsensing of Road Pavements Conditions (Delnevo et al., 2025) ShakeSensing is a mobile phone application that enables phone sensors (accelerometers and GPS) to measure vibrations when a cyclist or scooter rider experiences coarse roads. The collected data is sent to a central server and displayed on maps, helping both city authorities and users. The system is affordable and easy to deploy because it uses devices that people already own. However, it faces problems such as inconsistent data due to different phone models, sensor positions (e.g., pockets or mounted locations), and privacy concerns. The system also depends on active user participation, which may not be feasible in all cities [3].

- RoadSense: A Framework for Road Condition Monitoring using Sensors and Machine Learning (Jahan et al., 2024) RoadSense is a low-cost and compact device designed for real-time road condition monitoring. It uses IMU sensors (MPU-6050), GPS, and environmental sensors to collect important data and identify problems such as potholes, speed breakers, and sudden braking. The device applies machine learning models, specifically tree-based ensemble methods, to process the collected data and detect road anomalies with an accuracy of 97% under different conditions. However, it also has some limitations. Its performance depends on the quality of the training data and the installed sensors. In addition, the system has only been tested in limited environments, so it may not perform well in unpredictable or highly diverse situations. Despite these limitations, its low cost and scalability make it a promising solution for large- scale road monitoring [4].

- Hybrid   Vehicle-to-X   Communication (Stellwagen   et   al.,   2023) This study looks at how hazardous information can be shared between the vehicles and with roadside systems to make traffic safer.  It integrates two systems of communication, ITS-G5 (a local communication in Wi-Fi) and LTE-V2X (cellular) to create a hybrid system that provides guaranteed communication. Its key benefit is the backup. Even if one method stops working, the other comes through to ensure transmission of the hazard warnings.

Although this system helps to manage the traffic and make everyone safer, this system alone can’t detect hazards like potholes. It instead depends on the other sensors or systems to identify those dangers first. Once the hazard is found, the system then shares information among the vehicles and roadside infrastructure so drivers can be warned and safety for everyone can be ensured [5].

- The Pothole Patrol: Using a Mobile Sensor Network for Road Surface Monitoring (Eriksson	et	al.,	2008) The project entailed the use of tracking devices and accelerometers attached to ordinary cars to identify bump hazards on the road. The system registered the vibration when a car collided with a pothole and its position was mapped. The data obtained was forwarded to a central server to generate a map of potholes in the city. This method was cost effective and scalable but at times mixed potholes with other bumps and there was no mechanism of determining whether a pothole was repaired.[6].

### Summary of Shortcomings

- ShakeSensing is cheap and scalable, but it depends on user involvement and lacks consistency in data.

- RoadSense is both affordable and accurate, but it is tested in limited areas and needs calibration.

- Hybrid V2X gives better reliability to messages though not detection.

- The Pothole Patrol is cost-efficient, inexpensive, and not very precise.

### Relevance to Our Project

Such solutions indicate improvement but also shed light on certain gaps. The sensors will be low-cost (accelerator, gyroscope, GPS) and the firebase verification will be employed to offer a more practical, affordable, and reliable road monitoring system.

It is possible to fill these gaps by the proposed work by means of mounting orientation calibration, using more than one sensor type (accelerator + gyroscope + speed) as well as optimizing detection algorithms on embedded hardware.

## Business Scope

This project is aimed at offering a real time and scalable solution to road monitoring at a low cost. From a business perspective, it targets the government, transport agencies and other companies who focus on the improvement of road infrastructure and ensure safety.

### Primary Target Market

Drivers and Vehicle owners: By providing real-time information regarding road conditions, it will help plan safer and more comfortable routes hence reducing possible wear and tear of a vehicle, saving future maintenance costs [7].

Ridesharing and Delivery Company drivers: This service allows the ridesharing and delivery drivers to select routes which reduce vehicular damage and discomfort, enhancing their job satisfaction [7].

Fleet Managers & Company Owners: The system’s data can be used by them to improve their routes, save expenses and reduce vehicle damage. Hence, things can be performed more efficiently and in a cost-effective manner. Companies that build autonomous vehicles have also started to implement this system for communication among them [8].

### Secondary Target Market

Local Government: The system provides the government with a better way of identifying and prioritizing the roads that require maintenance [8].

Civil Engineers & Urban Planners: They use this service as a source of long-term data for analyzing and evaluating infrastructure which helps them to make decisions based on data for future projects [7].

Mapping and Navigation Companies: These companies may integrate this service to be able to access live data regarding road conditions, hence offering their users more accurate and better routing information [7].

### Additional Market Potential

The proposed technology provides a commercially scalable solution for fleet operators, municipal governments, and business sectors through models that are subscription based.

It can be used by the public and private companies of the transport, logistics, and road maintenance industry due to its adaptability to urban and industrial environment. [8].

### Resources Needed for Scaling

To commercialize this solution, the team will have to:

Cover more areas with sensors through collaborations with vehicle fleets, local governments and other entities.

- Invest in a good data infrastructure to support massive monitoring and analysis.

- Determine marketing and partnership strategies to target governments, companies, and technology companies.

- Keep optimizing machine learning models to provide more accurate predictions that can also identify anomalies, which are reliable and effective in the long term.

To make the roadmap stronger and to keep the business strategy to continue the right path, a lean canvas has been prepared (Table 1-1). It helps give a clear and a more structured view of all the important elements that helps in guiding towards the commercialization of the solution.

Table 1-1: Lean Canvas for RoadBounce

Problem; Solution; Unique Value Proposition; Unfair Advantage; Customer Segments. 1. Roads in Pakistan suffer from frequent potholes and poor maintenance. 2. Current monitoring is manual, slow, and costly. 3. Apps like Waze depend on manual reporting, which is unreliable.; 1. Low-cost embedded device (accelerometer + gyroscope + GPS) to auto-detect potholes. 2. Data uploaded to a central server that aggregates and verifies anomalies. 3. Mobile/web dashboard showing updated pothole maps for authorities and drivers.; "An affordable, automatic, and scalable road monitoring system that detects potholes in real time without manual reporting."; 1. Sensor-based automation — no user input needed, unlike Waze. 2. Can be deployed on any vehicle, not just luxury cars. 3. Adapted for local conditions (Pakistan roads), more relevant than foreign products.; 1. Everyday drivers (navigation apps with pothole alerts). 2. Government authorities & municipalities (road maintenance planning). 3. Transport companies / logistics fleets (avoid vehicle damage).. Existing Alternatives; Key Metrics; High-Level Concept; Channels; Early Adopters. 1. Manual inspection (slow, expensive). 2. Waze / Google Maps hazard reports (manual, crowdsourced, often inaccurate). 3. Premium car features e.g. Mercedes Car-to-X (not accessible in Pakistan).; 1. Potholes detected per day/week. 2. Detection accuracy (true potholes vs. false bumps). 3. Active users (drivers, vehicles installed). 4. Reports used by authorities to repair roads.; Think of it as Waze for potholes — but automatic and sensor-based instead of manual.; 1. Partnerships with ride-hailing services, bus fleets, courier/logistics companies. 2. University / research pilots in smart city programs. 3. Pilot projects with city governments.; 1. Tech-savvy drivers who use navigation apps daily. 2. Municipal authorities looking for smart, low-cost city solutions. 3. Fleet operators (delivery, buses) wanting to reduce vehicle wear & tear.. Cost Structure; Revenue Structure. 1. Hardware: sensors, embedded devices (Raspberry Pi), GPS modules. 2. Firebase database costs. 3. Mobile app development. 4. Marketing and partnerships.; 1. B2G: Subscription/licensing for municipalities to access pothole maps. 2. B2B: Fleet dashboard & maintenance alerts. 3. Freemium: Free navigation app, premium analytics for businesses. 4. Data services: Selling Road quality data to insurers, urban planners, delivery platforms..

## Useful Tools and Technologies

Several important parts contribute to the building of the road surface monitoring system, and each one helps in achieving its goals. Following is a list of some primary tools that are used with their functions:

### Useful Hardware

Hardware and some other key parts utilized in building the system are explained in this section.

#### Accelerometer

To identify any anomaly, a road monitoring system's accelerometer monitors the accelerations and vibrations that are produced by a moving car that is shown below in Figure 1-1 [9].

Figure 1-1: Accelerometer Sensor

#### Gyroscope

A gyroscope in a road monitoring system analyses a vehicle's angular velocity and orientation changes, allowing it to detect road surface defects using rotational movement data. This complements accelerometer data in providing an expanded view of road conditions as shown in Figure 1-2 [10].

Figure 1-2: Gyroscope Sensor

The system will use MPU-6050, which combines 3-axis accelerometer and 3-axis gyroscope that is shown in Figure 1-3.

Figure 1-3: MPU-6050

#### GPS

GPS (Global Positioning System) is a satellite navigation technology that collects signals from many satellites to deliver real-time location, speed, and time data. It allows for exact tracking and mapping of cars and road conditions for effective navigation, monitoring, and traffic management. Due to high accuracy and efficiency, the system uses NEO-M8N GPS Module with Serial UART Interface which is shown in Figure 1-4 [11].

Figure 1-4: NEO-M8N GPS module

#### Single-Board Computer

A single board computer (SBC) is a small, integrated computer designed on a single circuit board, and contains essential components such as the processor, memory, and input/output interfaces. It will be used to gather sensor data, process it in real-time within the vehicle, and transmit any detected road irregularity to a central system. This allows for effective road surface monitoring without needing any large equipment. The system will Raspberry- Pi 4 to run complex machine learning algorithms directly on the device for highly accurate, real-time analysis of sensor data shown in Figure 1-5.

Figure 1-5: Raspberry-Pi 4 SBC

#### 12v/ 5 amp battery

A 12V rechargeable battery is used as the primary power source for the system. It provides portable and reliable power to the Raspberry Pi and other connected components, allowing the project to operate independently without a direct electrical outlet. The battery is suitable for mobile and outdoor applications due to its stable voltage output and rechargeable capability shown in Figure 1-6.

Figure 1-6: 12v/ 5 amp battery

#### Step Down Voltage Regulator

The XL4016 Step-Down Voltage Regulator is used to convert the 12V output from the battery into a stable 5V supply required by the Raspberry Pi. It is a high-efficiency DC-DC buck converter capable of handling higher current loads with minimal power loss. The regulator helps protect the Raspberry Pi from voltage fluctuations and ensures smooth and stable system operation shown in Figure 1-7 [12].

Figure 1-7: Step Down Voltage Regulator

### Useful Software

This section explains the software, programming languages, and tools used to develop and test the road monitoring system.

#### Development Environment

The mobile application is created with Android Studio, the official Integrated Development Environment (IDE) for Android app development. The app's key functions include receiving processed road data from the hardware device and displaying essential information to the user (for example, the position of anomalies on a map). It also serves as a secure communication gateway, allowing data transmission shown in Figure 1-8.

Figure 1-8: Android Studio Icon

Google Colab is a cloud-based environment that is free to use and allows users to execute and write scripts in python on their browser shown in Figure 1-9 [13].

Figure 1-9: Google Colab Icon

Firebase is clouding hosted NoSQL Database from Google that allows developers to store and synchronize data in real-time across devices. It eliminates the need for managing traditional server-side infrastructure, offering a serverless solution that updates connected clients instantly shown in Figure 1-10.

Figure 1-10: Firebase Icon

#### Programming Languages

Python is a powerful and flexible programming language, known for its easy-to-understand syntax and a huge range of libraries. It is widely used in the field of artificial intelligence shown in Figure 1-11 .

Figure 1-11: Python Icon

Kotlin is a modern, concise programming language developed by JetBrains. It's fully interoperable with Java and popular for Android development, server-side apps, and multiplatform projects shown in Figure 1-12 [14].

Figure 1-12: Kotlin Icon

XML (Extensible Markup Language) is a markup language similar to HTML, but without predefined tags to use. Instead, you define your own tags designed specifically for your needs. This is a powerful way to store data in a format that can be stored, searched, and shared shown in Figure 1-13 [15].

Figure 1-13: XML Icon

## Project Work Break Down

Figure 1-14 below explains the work breakdown structure of the RoadBounce Road Monitoring System project. The project is divided into different phases, including research, hardware and software development, integration, testing, and final deployment. Each phase contains important tasks needed to complete the project successfully. The tasks are divided among group members, which are indicated by background colours with purple for Farhan Ali, blue for Tooba Fatima, and Orange for Laiba Taj to show individual responsibilities.

Figure 1-14: Work-Breakdown Structure

## Project Timeline

Figure 1-15 outlines the timeline for the RoadBounce Road Monitoring System project, running from September 2025 to May 2026. The project started with the Research and Planning phase, which will continue through October 2025. Following that, two key stages will overlap: Hardware Development from November 2025 to February 2026, and Software Development from December 2025 to February 2026. During Hardware Development, the focus will be on integrating parts such as the accelerometer, microprocessor, and dashboard display into the vehicle. At the same time, the Software Development phase will concentrate on creating the mobile app and the central data server. From February to March 2026, the Integration and Communication phase will bring everything together, ensuring that the hardware is able to communicate with the server for data transmission. Finally, the system will undergo extensive Testing and Validation from mid-March to April 2026 to ensure everything is working well. The project wraps up with the Deployment and Documentation phase, starting in mid-April 2026 and continuing until the end of May 2026.

Figure 1-15: Project Timeline Gantt Chart.

# Chapter 2

# Requirement Specification and Analysis

This chapter covers the requirements and the functionality of the RoadBounce system. The functional requirements, such as road anomaly detection, data logging, and the display of results are covered. The non-functional requirements focus on how the system should behave, which includes security, speed, accuracy, and ease of maintenance. Thus, it describes the basis for developing a system that is effective at every layer, from collecting raw sensor data to delivering a functioning application for the users of the app [16]. The system architecture consists of:

- Vehicle Sensor Unit (VSU).

- Firebase Database which will receive, process and validate road anomaly data from multiple vehicles.

- Mobile Application users can view live road condition maps and reports.

## Functional Requirements

The basic functions of the RoadBounce system are described by the functional requirements. Each requirement has been classified as either Core, Supportive or optional shown in Table 2-1.

Table 2-1: Functional Requirements for RoadBounce

Data Acquisition

| ID | Name | Description | Type |
| --- | --- | --- | --- |
| FR1 | Sensor Data Collection | The system shall collect real-time sensor data (accelerometer, gyroscope, and GPS) continuously while the vehicle is in motion. | Core |
| FR2 | GPS Data Tracking | The system shall include GPS position (latitude, longitude), speed, and time when a sensor measures data and then plots the anomalies. | Core |
| FR3 | Sensor Calibration | The system shall first calibrate the sensors (i.e. noise filtering) before starting collection of data so that it is precise. | Supportive |
| FR4 | Environmental Adaptation | The system shall filter out false alerts caused by uphill or downhill slopes by adjusting sensor data. It shall not raise false alarms when vehicle is at slower speeds or for engine vibrations, but only for high speeds. | Core |

Data Cleaning and Filtering

| FR5 | Bandpass Filter Application | The system shall use Bandpass Filtering of sensor data to separate the signal of road-induced vibration by eliminating low-frequency vehicle body motion and high-frequency sensor noise. | Core |
| --- | --- | --- | --- |
| FR6 | Road Condition Classification | To detect anomalies, the system shall run a trained machine learning model (1D CNN) on the filtered sensor data and categorize it accordingly. | Core |

Road Anomaly Detection

| FR7 | Road Vibration Analysis | The system shall process accelerometer and gyroscope data to recognize patterns that indicate a road anomaly using vibration thresholds. | Core |
| --- | --- | --- | --- |
| FR8 | Anomaly Marking and Storage | The system shall mark each identified road anomaly with GPS coordinates in real time and store the data locally for each anomaly. This data will then be transferred to Firebase once network connectivity is available. | Core |

Communication and Data Handling

| FR9 | Data Transmission | The system shall send collected data to the central server via Wi-Fi regularly. | Core |
| --- | --- | --- | --- |

Server-Side Processing and Management

| FR10 | Data Reception & Storage | The Firebase shall receive and store incoming information from vehicles in a standardized format. | Core |
| --- | --- | --- | --- |
| FR11 | Data Validation | The Firebase shall ensure that incoming data has been validated as being correct, then it will be processed. | Core |
| FR12 | Anomaly Clustering | The system shall group repeated detections of the same anomaly as a single, when detected by more than one vehicle. | Core |
| FR13 | Road Map Generation | The Firebase shall generate and constantly update a graphical map of road conditions represented as color-coded pins, based on the nature of anomaly. | Core |

Mobile Application

| FR14 | Account Login and Authentication | The mobile app shall require secure login and passwords for authorized access to road data and analytics. | Core |
| --- | --- | --- | --- |
| FR15 | Real-Time Map Display | The mobile app shall display a live map of road defects detected with their locations and types. | Core |
| FR16 | Notification Alerts | The system shall alert drivers or authorities when critical road defects are detected on regular or habitual routes. | Supportive |
| FR17 | Best Route Display | The mobile app shall display the best possible route for the user, considering the real-time road conditions. | Core |

Administrative & Integration

| FR18 | Multi-Vehicle Support | The Firebase shall serve simultaneous data records of multiple vehicles without loss or duplication of data. | Core |
| --- | --- | --- | --- |

Note: Adapted from IoT road monitoring systems [16] [17].

## Non-Functional Requirements

The non-functional requirements (NFRs) include performance, quality, and constraints that the RoadBounce system should meet to be efficient, secure, and operate on scale. Each requirement is given a priority rank - High, Medium, or Low shown in Table 2-2.

Table 2-2: Non-Functional Requirements for RoadBounce

| ID | Category | Description | Priority |
| --- | --- | --- | --- |
| NFR1 | Security | The system shall use AES or HTTPS encryption for communication between the Vehicle Sensor Unit and the Central Server to ensure data integrity and confidentiality. | High |
| NFR2 | Accuracy | The detection algorithm shall have an accuracy of at least 80 to 90% in recognizing the anomalies under standard conditions. | High |
| NFR3 | Performance | The VSU shall quickly run the entire process of detecting road anomalies ensuring that the detection results are produced almost immediately to reduce delay in reporting. | High |
| NFR4 | Availability (Server) | The Firebase Database shall maintain a minimum service availability (uptime) of 99.0% to ensure reliable access for all users and VSUs. | High |
| NFR5 | Reliability | The VSU shall maintain data integrity even when network is unstable by locally storing anomaly reports and transmitting them later once the connection is reliable, which ensures no data is lost or missed when network conditions are inconsistent. | High |
| NFR6 | Scalability | The Firebase shall support multiple connected vehicles all transmitting data simultaneously and without degradation. | High |
| NFR7 | Interoperability | The system shall interface with mapping tools, e.g. Google Maps. | Medium |
| NFR8 | Maintainability | The code will be modular and well documented, making it easy to troubleshoot and upgrade in the future. | High |
| NFR9 | Usability | The mobile app and dashboard interfaces shall be easy to use for the non-technical user to access road data; they will be intuitive and user-friendly. | High |
| NFR10 | Data Privacy | The system shall anonymize all user identifiers and vehicle identifiers before uploading them to the central database. | High |

Note: Non-functional requirements adapted from IoT road monitoring systems [16].

## Epics

An epic is a big project or a goal that has been broken down into smaller and more manageable pieces called user stories. These can span multiple teams and take several sprints to complete, which means the scope can evolve as feedback comes in from customers or as the team makes progress. Epics help organize complex work by linking everyday tasks to broader business objectives, offering a clear picture of what's being worked on. They're usually planned to last a few weeks, making large projects easier to handle while continuously delivering value. This structure helps keep everyone aligned and focused on achieving the end goal [18].

### Epic 1

E1: Hardware Integration

Description: As a hardware engineer, I want to integrate and calibrate accelerometer, gyroscope, and GPS hardware into a single system so that accurate and synchronized data of the road surface can be collected in real-time.

#### User stories

Table 2-3 displays all user stories, their description and acceptance criteria for epic 1.

Table 2-3: Epic 1 User Stories

| User story | Description | Acceptance criteria |
| --- | --- | --- |
| E1-US1: Sensor Data Collection | As a hardware engineer, I want the system to collect real-time sensor data from accelerometers, gyroscopes, and GPS continuously while the vehicle is in motion, so that accurate data can be used for road anomaly detection. | Given the vehicle is in motion, When the sensors are activated, Then the system collects accelerometer, gyroscope, and GPS data continuously. |
| E1-US2: Sensor Calibration | As a hardware engineer, I want the system to perform an initial calibration of the accelerometer and gyroscope sensors so that we can ensure accurate readings for data collection. | Given the sensor module is connected and ready for calibration, When the engineer initiates the calibration process for accelerometer and gyroscope sensors, Then the system completes calibration and displays calibrated readings that are accurate within predefined tolerance limits. |
| E1-US3: GPS Integration with Sensor Data | As a hardware engineer, I want GPS data to be integrated with sensor data so that the system can flag each anomaly with accurate GPS coordinates. | Given GPS and other sensors are active and collecting data, When a road anomaly is detected by the accelerometer or gyroscope, Then the system tags the anomaly with precise GPS coordinates and combines all relevant sensor data for that location. |

### Epic 2

E2: Data Processing and Anomaly Detection

Description: As a data scientist, I want to preprocess the sensor data and apply machine learning algorithms so that potholes and other road anomalies are correctly detected and classified in real-time.

#### User stories

Table 2-4 displays all user stories, their description and acceptance criteria for epic 2.

Table 2-4: Epic 2 User Stories

| User story | Description | Acceptance criteria |
| --- | --- | --- |
| E2-US1: Apply Bandpass Filtering to Sensor Data | As a data scientist, I want to apply a bandpass filter to sensor data so that road- induced vibration signals are isolated by removing low-frequency body movement and high- frequency sensor noise. | Given raw accelerometer and gyroscope data, When the bandpass filter is applied, Then the filter isolates the road-induced vibrations while removing unwanted noise from body movement and sensor noise. |
| E2-US2: Classify Road Anomalies with ML | As a data scientist, I want to apply machine learning algorithms to classify road anomalies so that potholes and cracks are detected with a high level of accuracy. | Given pre-processed sensor data, When the machine learning model classifies anomalies, Then potholes and cracks are detected with an accuracy between 80% and 90%. |
| E2-US3: Real-time Road Anomaly Detection | As a data scientist, I want the system to detect road anomalies in real-time so that immediate action can be taken when hazards are identified. | Given continuous sensor data input, When an anomaly occurs, Then the system detects and reports the anomaly in real- time with minimal delay. |
| E2-US4: Group Repeated Anomaly Detections into a Single Entry | As a system, I want to group repeated detections of the same road anomaly from multiple vehicles into a single anomaly record, so that the system can avoid duplication and maintain accurate data about road conditions. | Given multiple vehicles have detected the same road anomaly, When the system processes the data from the vehicles, Then it should group these detections into a single anomaly record rather than creating separate entries. |
| E2-US5: Environmental Adaptation for Accurate Anomaly Detection | As a data scientist, I want the system to ignore false alerts caused by uphill or downhill slopes and adjust sensor data so that potholes are only detected when the vehicle moves faster than a specified speed limit, preventing false alarms. | Given the vehicle is on incline or moving slowly, When the system processes sensor data, Then it filters out false anomalies caused by slopes and low-speed vibrations. |
| E2-US6: Record and Store Road Anomalies with GPS Coordinates | As a system, I want to record and store each identified road anomaly with GPS coordinates locally, so that it can be transferred to the server later when a network connection is available. | Given that a road anomaly is detected, When the anomaly is identified, Then the anomaly is recorded and stored locally with GPS coordinates. |
| ES2-US7: Mark Road Anomalies in Real-Time | As a system, I want to mark detected road anomalies with GPS coordinates and store them locally until they can be transferred to the server. | Given a road anomaly is detected, When the anomaly is identified, Then the system marks the anomaly with GPS coordinates and stores it locally. |
| E2-US8: Support Data from Multiple Vehicles Simultaneously | As a system administrator, I want Firebase to handle data streams from multiple vehicles simultaneously, ensuring no data loss or duplication. | Given multiple vehicles sending data to the server, When data is processed, Then no data loss or duplication occurs. |

### Epic 3

E3: Mobile Application Development and User Experience

Description: As a driver, I want a mobile app with real-time road condition maps, alerts, route suggestions, and an intuitive interface so that I can plan safer, more efficient routes.

#### User stories

Table 2-5 displays all user stories, their description and acceptance criteria for epic 3.

Table 2-5: Epic 3 User Stories

| User story | Description | Acceptance criteria |
| --- | --- | --- |
| E3-US1: Secure Login for Mobile Application | As a user, I want the mobile app to require secure login and password authentication to ensure authorized access to road data. | Given that the mobile app is open, When the user enters their credentials, Then the app ensures that the user is authenticated before granting access. |
| E3-US2: Display Road Conditions on Map | As a user, I want to see marked road conditions on a map so that I can avoid roads with anomalies and plan better routes. | Given processed anomaly data, When a driver views the map app, Then roads are visually differentiated between good conditions and those with anomalies to facilitate route choice. |
| E3-US3: Real-time Alerts for Users | As a user, I want to receive real-time alerts about road anomalies so that I can reroute vehicles and avoid dangerous areas. | Given monitored routes with active sensors, When a severe road anomaly is detected, Then the user receives an immediate alert to enable rerouting. |
| E3-US4: Display the Best Route Based on Real- Time Conditions | As a mobile app user, I want the app to display the best possible route based on real-time road conditions, so I can avoid hazards and take the most efficient route. | Given that real-time data is available, When the user inputs a destination, Then the app calculates and displays the best route considering the road conditions. |
| E3-US5: Intuitive Mobile App Interface | As a mobile app user, I want the interface to be clean, intuitive, and easy to navigate, so that I can use the app effectively without requiring extensive training. | Given the mobile app is opened, When the user navigates through the app, Then the app should be intuitive, with clear icons, minimal steps, and a well- organized layout that allows users to easily view and interact with road data. |
| E3-US6: Display List of History of Trips | As a mobile app user, I want to see a list of my previous trips in the app so that I can review them and revisit important details. | Given the user is logged into the app, When the user navigates to the " Routes History" section, Then the app should display a list of all the user's previous trips, showing relevant information. |
| E3-US7: Display Anomaly Info Card | As a user, I want to see detailed information about a detected road anomaly in an info card so that I can understand the severity, location, and other key details of the anomaly. | Given that the user has clicked on an anomaly marker on the map, When the anomaly info card is displayed, Then the card should show detailed information such as the anomaly type, location, severity. |

### Epic 4

E4: System Integration and Secure Communication

Description: As a system integrator, I want reliable and secure transmission of data between vehicle sensor units and Firebase so that data integrity is maintained and we have timely reporting.

#### User stories

Table 2-6 displays all user stories, their description and acceptance criteria for epic 4.

Table 2-6: Epic 4 User Stories

| User story | Description | Acceptance criteria |
| --- | --- | --- |
| E4-US1: Secure, Encrypted Communication | As a system integrator, I want all data transmitted between vehicle units and Firebase to be encrypted to ensure security and privacy. | Given vehicle units and server communication channels, When data is transmitted, Then all data is encrypted and verified to protect privacy and ensure integrity. |
| E4-US2: Real-time Data Synchronization | As a system integrator, I want the system to synchronize data in real- time so that there are minimal delays in anomaly reporting. | Given multiple data sources streaming simultaneously, When data is collected and processed, Then synchronization happens in near real-time to minimize delays in anomaly reporting. |
| E4-US3: Receive and Store Data on the Central Server | As a Firebase administrator, I want the system to receive and store incoming data from vehicles in a standardized format to maintain consistency. | Given incoming data from vehicles, When the data is received by the Firebase, Then the data is stored in a standardized format. |

### Epic 5

E5: Testing, Validation, and Deployment

Description: As a QA engineer, I want to test system components thoroughly, across varied conditions so that performance and reliability can be ensured before deployment.

#### User stories

Table 2-7 displays all user stories, their description and acceptance criteria for epic 5.

Table 2-7: Epic 5 User Stories

| User story | Description | Acceptance criteria |
| --- | --- | --- |
| E5-US1: Test Sensor Accuracy on Different Roads | As a QA engineer, I want to test sensor accuracy across different road types to ensure they perform well in varied environments. | Given different road types in testing environments, When sensor readings are collected, Then sensor accuracy is validated against ground truth standards for each road type. |

Note: User stories based on established IoT road monitoring architecture [16].

## Test-cases

Test Cases are defined scenarios that describe the specific inputs, execution conditions, and expected outcomes used to verify whether a particular feature of the system works as intended. They help ensure that requirements are met, identify defects, and validate the overall quality of the software [19]. Test cases based on IoT road monitoring systems [16].

### Test Case 1: Continuous Sensor Data Collection

Table 2-8 illustrates how sensor data is continuously collected from the hardware module during vehicle movement.

Table 2-8: Continuous Sensor Data Collection

| Test ID: E1-US1-TC1 | US ID: E1-US1 |
| --- | --- |
| Test Case Description: This test case verifies that real-time sensor data is collected continuously from accelerometers, gyroscopes, and GPS while the vehicle is in motion. |  |
| Inputs: - Accelerometer, gyroscope, and GPS sensors active - Vehicle in motion |  |
| Expected Result: - Sensor data is collected continuously without interruption while the vehicle is in motion. |  |

### Test Case 2: Sensor Calibration

Table 2-9 presents the sensor calibration process performed before real-time data acquisition. Calibration is necessary to reduce sensor noise, correct offsets, and improve the accuracy of accelerometer readings used for road anomaly detection.

Table 2-9: Sensor Calibration

| Test ID: E1-US2-TC1 | US ID: E1-US2 |
| --- | --- |
| Test Case Description: This test case verifies successful completion of accelerometer and gyroscope calibration process and ensures the system provides accurate readings within predefined limits of tolerance. |  |
| Inputs: - Accelerometer and gyroscope sensor module connected - Calibration process triggered manually by the hardware engineer - Predefined tolerance limits for calibration |  |
| Expected Result: - Calibration is completed successfully - The system displays the calibrated readings that are accurate within the predefined tolerance limits |  |

### Test Case 3: GPS Integration with Sensor Data

Table 2-10 describes the integration of GPS data with sensor readings. GPS coordinates are combined with accelerometer data to associate detected road anomalies with their exact geographical locations.

Table 2-10: GPS Integration with Sensor Data

| Test ID: E1-US3-TC1 | US ID: E1-US3 |
| --- | --- |
| Test Case Description: This test case ensures that GPS data is accurately tagged with road anomalies, and that the system combines sensor data with GPS coordinates for correct identification of anomalies. |  |
| Inputs: - GPS and other sensors (accelerometer, gyroscope) are actively collecting data - A road anomaly is detected (e.g., pothole or bump) - GPS coordinates of the current location |  |
| Expected Result: - The system tags the detected anomaly with precise GPS coordinates - The sensor data for that anomaly is correctly combined with the GPS data |  |

### Test Case 4: Bandpass Filter for Sensor Data

Table 2-11 presents the application of a bandpass filter on sensor data to remove unwanted noise and retain the frequency range associated with road anomalies. This filtering process improves the quality and reliability of the accelerometer signals used for pothole detection.

Table 2-11: Bandpass Filter for Sensor Data

| Test ID: E2-US1-TC1 | US ID: E2-US1 |
| --- | --- |
| Test Case Description: This test case verifies that the bandpass filter isolates road- induced vibrations by removing low and high-frequency noise. |  |
| Inputs: - Raw accelerometer and gyroscope data - Bandpass filter algorithm |  |
| Expected Result: - The filtered data shows clear road vibrations, with minimal noise from vehicle body movement and sensor noise. |  |

### Test Case 5: Classify Road Anomalies with Machine Learning

Table 2-12 describes the machine learning-based classification process used to identify road anomalies from processed sensor data. The trained model analyses extracted features to detect anomalies based on severity and road conditions.

Table 2-12: Classify Road Anomalies with Machine Learning

| Test ID: E2-US2-TC1 | US ID: E2-US2 |
| --- | --- |
| Test Case Description: This test case validates that the 1d cnn model classifies road anomalies accurately (with 80% accuracy), including potholes, bumps, and cracks. |  |
| Inputs: - Pre-processed sensor data (accelerometer and gyroscope data) - Trained machine learning model for anomaly classification |  |
| Expected Result: 1. Road anomalies are detected and classified with 80% accuracy |  |

### Test Case 6: Real-time Road Anomaly Detection

Table 2-13 presents the real-time road anomaly detection process implemented in the proposed system. Sensor data collected from the vehicle is processed and analysed continuously by the machine learning model to identify road anomalies and generate detection results in real time.

Table 2-13: Real-time Road Anomaly Detection

| Test ID: E2-US3-TC1 | US ID: E2-US3 |
| --- | --- |
| Test Case Description: This test case ensures that the system detects road anomalies in real-time, with minimal delay immediately after they occur. |  |
| Inputs: - Continuous sensor data input (accelerometer and gyroscope) - Real-time anomaly detection algorithm |  |
| Expected Result: - The system detects the road anomaly immediately after it occurs - The anomaly is reported in real-time, with minimal delay |  |

### Test Case 7: Clustering of Repeated Anomaly Detections

The following table 2-14 describes the clustering process used to group repeated detections of the same road anomaly and avoid duplicate reporting.

Table 2-14: Clustering of Repeated Anomaly Detections

| Test ID: E2-US4-TC1 | US ID: E2-US4 |
| --- | --- |
| Test Case Description: This test case ensures that the system groups repeated detections of the same anomaly by multiple vehicles as a single anomaly. |  |
| Inputs: - Anomaly detection data from multiple vehicles (e.g., potholes, bumps) - GPS coordinates and timestamps from each detection |  |
| Expected Result: - The system clusters the detections into one anomaly record with the combined data from all vehicles. - No duplicate records are created for the same anomaly. |  |

### Test Case 8: False Alert Filtering for Environmental Conditions

Table 2-15 presents the false alert filtering mechanism used to reduce incorrect detections caused by environmental conditions and sensor noise.

Table 2-15: False Alert Filtering for Environmental Conditions

| Test ID: E2-US5-TC1 | US ID: E2-US5 |
| --- | --- |
| Test Case Description: This test case verifies that false alerts are filtered out for uphill or downhill slopes and slow vehicle speeds. |  |
| Inputs: - Sensor data with low vehicle speed or inclines - Speed threshold and slope detection algorithm |  |
| Expected Result: - The system does not trigger false pothole detection during slow or inclined vehicle movement. |  |

### Test Case 9: Road Anomaly Recording and Storage with GPS Coordinates

The following table 2-16 describes the process of recording detected road anomalies along with their GPS coordinates for storage and future analysis.

Table 2-16: Road Anomaly Recording and Storage with GPS Coordinates

| Test ID: E2-US6-TC1 | US ID: E2-US6 |
| --- | --- |
| Test Case Description: This test case verifies that the system records each identified road anomaly along with GPS coordinates and stores it locally for later transmission to the Firebase. |  |
| Inputs: 1. Detected road anomalies with GPS coordinates |  |
| Expected Result: 1. Each identified anomaly is recorded with the correct GPS coordinates. |  |

### Test Case 10: Marking of Road Anomalies with GPS Coordinate

Table 2-17 presents the process of marking detected road anomalies using GPS coordinates to identify their exact locations on the road network.

Table 2-17: Marking of Road Anomalies with GPS Coordinates

| Test ID: E2-US7-TC1 | US ID: E2-US7 |
| --- | --- |
| Test Case Description: This test case verifies that the system correctly marks each detected road anomaly with GPS coordinates and stores it locally until it can be transferred to the server. |  |
| Inputs: - Sensor data indicating a road anomaly (e.g., pothole, bump) - GPS corresponding to the detected anomaly - System to store the data locally |  |
| Expected Result: - The system should accurately mark the detected anomaly with the correct GPS coordinates. |  |

### Test Case 11: Prevent data loss/duplication in multi-vehicle streams

The following table 2-18 describes the mechanism used to prevent data loss and duplication when processing road anomaly reports from multiple vehicles simultaneously.

Table 2-18: Prevent data loss/duplication in multi-vehicle streams

| Test ID: E2-US8-TC1 | US ID: E2-US8 |
| --- | --- |
| Test Case Description: This test case verifies that the server can process multiple data streams from different vehicles simultaneously, ensuring no data loss or duplication. |  |
| Inputs: - Simultaneous data streams from multiple vehicles (e.g., road anomalies, GPS data, sensor readings) - Firebase receiving and processing data from each vehicle |  |
| Expected Result: - The Firebase should successfully process data from multiple vehicles at the same time without data loss or duplication. - Data from each vehicle should be properly separated and stored. - The Firebase should maintain data integrity, ensuring that each anomaly or sensor reading is correctly logged for its corresponding vehicle. - No duplication of data should occur in the database, even when data from multiple vehicles is being processed simultaneously. |  |

### Test Case 12: Secure Login for Mobile App

The following table 2-19 presents the secure login process implemented in the mobile application to ensure authenticated access and protect user data.

Table 2-19: Secure Login for Mobile App

| Test ID: E3-US1-TC1 | US ID: E3-US1 |
| --- | --- |
| Test Case Description: This test case ensures that the app requires secure login and password authentication before granting access to road data. |  |
| Inputs: 1. User credentials |  |
| Expected Result: 1. Only authorized users can access the app and its data. |  |

### Test Case 13: Display Road Conditions on Map

Table 2-20 describes the process of displaying detected road conditions and anomalies on the map interface for user visualization and navigation.

Table 2-20: Display Road Conditions on Map

| Test ID: E3-US2-TC1 | US ID: E3-US2 |
| --- | --- |
| Test Case Description: This test case verifies that the mobile app correctly displays the road conditions on the map, differentiating between roads in good condition and those with anomalies. |  |
| Inputs: - Processed anomaly data - Map rendering module in the mobile app |  |
| Expected Result: - The map visually differentiates roads in good condition and those with anomalies - Anomalies are marked clearly on the map for easier route planning |  |

### Test Case 14: Real-Time Alerts for Users on Road Anomalies

The following table 2-21 presents the real-time alert mechanism used to notify users about detected road anomalies and potential hazards during travel.

Table 2-21: Real-Time Alerts for Users on Road Anomalies

| Test ID: E3-US3-TC1 | US ID: E3-US3 |
| --- | --- |
| Test Case Description: This test case verifies that the mobile app sends real-time alerts to the user when severe road anomalies are detected, allowing the user to reroute the vehicle and avoid dangerous areas. |  |
| Inputs: - Monitored route with active sensors tracking real-time road conditions. - Severe road anomaly detected. |  |
| Expected Result: - The user receives an alert when a severe anomaly is detected along the route. |  |

### Test Case 15: Best Route Options Based on Real-Time Data

Table 2-22 describes the route recommendation process that uses real-time road condition data to suggest safer and more efficient travel routes.

Table 2-22: Best Route Options Based on Real-Time Data

| Test ID: E3-US4-TC1 | US ID: E3-US4 |
| --- | --- |
| Test Case Description: This test case verifies that the mobile app displays the best routes based on real-time road conditions. |  |
| Inputs: - Real-time road data (traffic, potholes, roadworks) - User destination input |  |
| Expected Result: - The best route is displayed based on the real-time conditions. |  |

### Test Case 16: Intuitive Mobile App Interface

Table 2-23 presents the user interface design features of the mobile application that provide a simple, intuitive, and user-friendly experience.

Table 2-23: Intuitive Mobile App Interface

| Test ID: E3-US5-TC1 | US ID: E3-US5 |
| --- | --- |
| Test Case Description: This test case ensures that the mobile app interface is clean, intuitive, and easy to navigate. |  |
| Inputs: - Mobile app interface - User interaction with the app |  |
| Expected Result: - The app is intuitive and easy to navigate - Clear icons and a well-organized layout for users to easily view and interact with road data |  |

### Test Case 17: List of History of Trips

The following table 2-24 describes the trip history feature that allows users to view and review their previously recorded journeys.

Table 2-24: List of History of Trips

| Test ID: E3-US6-TC1 | US ID: E3-US6 |
| --- | --- |
| Test Case Description: This test case ensures that the app correctly displays a list of the user’s past trips, including key details such as date, time, origin and destination location. |  |
| Inputs: - Valid user credentials - Data for multiple trips |  |
| Expected Result: - The "Routes History" section displays a list of trips with key details (trip date, destination). - The user can scroll through the list. - No trips are shown if the user has not completed any trips yet. |  |

### Test Case 18: Display Anomaly Info Card

Table 2-25 presents the anomaly information card used to display details of detected road anomalies, including their location, severity, and related information.

Table 2-25: Display Anomaly Info Card

| Test ID: E3-US7-TC1 | US ID: E3-US7 |
| --- | --- |
| Test Case Description: This test case ensures that when a user clicks on an anomaly marker on the map, an info card is displayed with detailed information about the anomaly. |  |
| Inputs: - Anomaly data with GPS coordinates and severity - User clicks on an anomaly marker on the map |  |
| Expected Result: - The app displays an info card with detailed information about the anomaly - The user can close the info card after viewing the details. |  |

### Test Case 19: Secure and Encrypted Communication

Table 2-26 presents the verification of secure and encrypted communication between vehicle sensor units and the central server. The test ensures that transmitted data remains protected from unauthorized access and maintains its integrity during transmission.

Table 2-26: Secure and Encrypted Communication

| Test ID: E4-US1-TC1 | US ID: E4-US1 |
| --- | --- |
| Test Case Description: This test case ensures that all transmitted data between vehicle sensor units and the central server is encrypted to protect privacy and maintain data integrity. |  |
| Inputs: - Vehicle sensor data - Encryption mechanism for data transmission - Communication channel between vehicle units and the server |  |
| Expected Result: - All data is encrypted during transmission - Data integrity is verified, and privacy is ensured |  |

### Test Case 20: Real-time Data Synchronization

Table 2-27 presents the testing of the real-time data synchronization mechanism. The test verifies that road anomaly data is transmitted and updated with minimal delay to ensure timely reporting.

Table 2-27: Real-time Data Synchronization

| Test ID: E4-US2-TC1 | US ID: E4-US2 |
| --- | --- |
| Test Case Description: This test case ensures that data synchronization occurs in real- time with minimal delay in anomaly reporting. |  |
| Inputs: - Multiple data sources streaming simultaneously - Data synchronization algorithm |  |
| Expected Result: - Data synchronization happens in near real-time - There is minimal delay in anomaly reporting |  |

### Test Case 21: Verify Data Reception and Standardized Storage

Table 2-28 presents the verification of data reception and standardized storage in Firebase. The test ensures that vehicle data is received correctly and stored in a consistent format for reliable processing and retrieval.

Table 2-28: Verify Data Reception and Standardized Storage

| Test ID: E4-US3-TC1 | US ID: E4-US3 |
| --- | --- |
| Test Case Description: Verify that the Firebase receives and stores vehicle data in a standardized format. |  |
| Inputs: 1. Incoming data from vehicles in various formats |  |
| Expected Result: 1. All data is stored in the correct standardized format. |  |

### Test Case 22: Test Sensor Accuracy on Different Roads

Table 2-29 presents the evaluation of sensor accuracy on different road surfaces. The test verifies the sensor's ability to collect reliable data across smooth, rough, and pothole-filled roads.

Table 2-29: Test Sensor Accuracy on Different Roads

| Test ID: E5-US1-TC1 | US ID: E5-US1 |
| --- | --- |
| Test Case Description: This test case ensures that sensor accuracy is tested across different road types (smooth, rough, pothole-filled). |  |
| Inputs: - Sensor readings from accelerometer and gyroscope on different road types. - Ground truth data for road conditions. |  |
| Expected Result: - Sensor readings are validated against standards for each road type. - The system ensures the sensors perform accurately across different road types. |  |

## User Interface Design (Prototypes)

These prototypes provide a clear visual representation of the application’s overall flow, core functionalities, and user interaction pathways. Each screen has been carefully designed to ensure intuitive navigation, seamless access to features, and efficient presentation of road condition data. These prototypes serve as the foundation for aligning user needs with system functionality and act as a bridge between conceptual requirements and the detailed traceability matrix that follows [16]. Navigation app patterns are followed as established by Google Maps [20].

### Onboarding Screen

The onboarding screen, shown in Figure 2-1,  welcomes users to the Road Bounce System with a friendly greeting and sets the theme of the application. A “Slide to Start” prompt encourages users to begin their journey smoothly and intuitively.

Figure 2-1: Onboarding Screen

### Sign-Up Page

The Sign-Up page as shown in Figure 2-2, collects user information such as name, email, phone, and password. The page contains text fields, a submit button, and validation messages for incorrect input. After successful signup, the user is redirected to the Login page.

Figure 2-2: Sign-up Page

### Sign-In Page

A Sign-In Page enables existing users to securely log in using their credentials, providing quick access to app features with a smooth and intuitive interface, which is shown below in Figure 2-3.

Figure 2-3: Sign-in Page

### User Dashboard (Map Display)

The user dashboard shows real-time road conditions on an interactive map, highlighting anomalies such as potholes, bumps, and cat’s eyes for easy navigation. Users can zoom, pan, and view detailed info about each road anomaly detected as shown in Figure 2-4.

Figure 2-4: User Dashboard (Map Display)

### Anomaly Info Pop-up Page

When a user taps on an anomaly on the map, a pop-up displays detailed information such as severity, location, and timestamp, helping users understand the road condition quickly which can be seen below in Figure 2-5.

Figure 2-5: Anomaly Info Pop-up Page

### Route Search page

Route searching allows users to enter a destination and view the best route on the interactive map, factoring in real-time road conditions and anomalies to suggest safer and smoother paths, shown in Figure 2-6 below.

Figure 2-6: Route Search Page

### Route Options Selection Page

Route option selection page displays all available routes for the entered destination, highlighting details like estimated time, distance, and road condition, allowing the user to select the most suitable route, as shown in Figure 2-7 below.

Figure 2-7: Route Options Selections Page

### Selected Route Map Display Page

Figure 2-8 shows the chosen route on the interactive map with real-time road conditions,

highlighting potholes, bumps, and cat’s eyes along the path to guide the user safely.

Figure 2-8: Selected Route Map Display Page

### User Profile Page

User profile page as shown in Figure 2-9 allows users to view their personal information, including name, email, password, and preferences, providing easy access to account management.

Figure 2-9: User Profile Page

### User History Page

Figure 2-10 shows a list of recently visited places, allowing the user to quickly revisit or select previously searched destinations. Provides a simple and clean view for history for improved navigation convenience.

Figure 2-10: User History Page

### Trip Summary Page

Figure 2-11 displays a detailed summary of a completed trip, including the route map, distance, duration, start/end points, and encountered anomalies. Also allows the user to quickly repeat the same route again with a single tap for hassle-free navigation.

Figure 2-11: Trip Summary Page

### Settings Drawer Page:

Figure 2-12 shows the settings page which provides quick access to app preferences such as language selection, notifications, terms & conditions, and support. This slide-out drawer enhances navigation by allowing users to adjust settings from anywhere in the app.

Figure 2-12: Settings Drawer Page

## Traceability Matrix

The following traceability matrix, Table 2-30, demonstrates how every epic, its user story and all relevant test cases are related, and which UI page corresponds with which epic. It ensures all requirements are verified during the testing process.

Table 2-30: Traceability Matrix

| Epic | User-Story | Test-case | UI |
| --- | --- | --- | --- |
| E1: Hardware Integration | E1-US1: Sensor Data Collection E1-US2: Sensor Calibration | E1-US1-TC1 E1-US2-TC1 E1-US3-TC1 . . | . . |
| E2: Data Processing and Anomaly Detection | E2-US1: Apply Bandpass Filtering to Sensor Data E2-US2: Classify Road Anomalies with ML E2-US3: Real-time Road Anomaly Detection E2-US4: Group Repeated Anomaly Detections into a Single Entry E2-US5: Environmental Adaptation for Accurate Anomaly Detection E2-US6: Record and Store Road Anomalies with GPS Coordinates E2-US7: Mark Road Anomalies in Real-Time E2-US8: Support Data from Multiple Vehicles Simultaneously | E2-US1-TC1 E2-US2-TC1 E2-US3-TC1 E2-US4-TC1 E2-US5-TC1 E2-US6-TC1 E2-US7-TC1 E2-US8-TC1 |  |
| E3: Mobile Application Development | E3-US1: Display Road Conditions on Map E3-US2: Real-time Alerts for Users E3-US3: Provide Real-time Alerts Based on Detected Conditions E3-US4: Display Best Route According to Real-time Traffic and Road Conditions E3-US5: Intuitive Mobile App Interface E3-US6: Display List of History of Trips E3-US7: Display Anomaly Info Card | E3-US1-TC1 E3-US2-TC1 E3-US3-TC1 E3-US4-TC1 E3-US6-TC1 E3-US7-TC1 | E3-US1-UI-4: User Dashboard (Map Display) E3-US1-UI-6: Route Search Page E3-US1-UI-7: Route Options Selection Page E3-US1-UI-8: Selected Route Map Display Page E3-US5-UI-1: Onboarding Screen E3-US5-UI-2: Sign-Up Page E3-US5-UI-3: Sign-In Page E3-US5-UI-9: User Profile Page E3-US5-UI-11: Route Selection from History Page E3-US5-UI-12: Settings Drawer Page E3-US6-UI-1: History Page E3-US7-UI-1: Anomaly Info Pop-up |
| E4: System Integration & Secure Communication | E4-US1: Secure, Encrypted Communication E4-US2: Real-time Data Synchronization E4-US3: Receive and Store Data on the Central Server | E4-US1-TC1 E4-US2-TC1 E4-US3-TC1 |  |
| E5: Testing, Validation & Deployment | E5-US1: Test Sensor Accuracy on Different Roads | E5-US1-TC1 |  |

# Chapter 3

# System Design

This chapter describes the implementation plan of the RoadBounce system linking the functionality requirements to their implementation. How the system’s key components such as sensors, data processing modules, and the user interface are structured and how they collaborate with each other to achieve the goals of the system is explained. The design will guarantee that all the components are scalable, reliable, and highly integrated to ensure that the requirements of the system are addressed. The objective is to establish a system that is capable of not only detecting and tracking abnormalities in the road but also gives useful information to the users to navigate the road safely and efficiently maintain the roads.

Detailed descriptions of software architecture, hardware specifications, communication protocols, data models, and workflow diagrams are included that ensure each part of the system supports functionality of the overall system. Real time data processing, efficient communication, and an intuitive user interface are prioritized by the proposed design such that the needs of all stakeholders are met, from drivers to government authorities.

## Software architecture

The RoadBounce system uses a hybrid approach for software architecture, which integrates Client-Server pattern for the overall structure, and Pipe-Filter pattern for the Vehicle Sensor Unit (VSU). This methodology gives a good blend of scalability, real-time processing and implementation which fits the project requirements.

### Client Server

The Client Server architecture is chosen as the main pattern as it supports the distributed nature of the system.

The VSU and the mobile application function as the client, while the central server receives and aggregates data from multiple vehicles. It clusters repeated anomaly reports, validates their detections, and then displays processed information to the application users [21].

Benefits:

      - It is easily scalable by the addition of more client vehicles without any changes needed to the server.

      - The centralized processing ensures that clustering done is dependable and removes any duplicate entries on the map.

      - In the absence of network, it supports automatic data backup upon restoration of network.

### Pipe-Filter

The information of each vehicle is handled through Pipe-Filter architecture. Each stage has a sequence of data flow [22].

Benefits:

      - The data passes through steps in a sequence and hence the process is easy to track.

      - Every step is independent, which makes it easier to update or modify the system without affecting it.

      - Any new filter may be added, and the system may expand without having to redesign.

      - In case any issue arises, it is easy to pinpoint which stage causes it making debugging more efficient.

## Components and Connectors

The RoadBounce system bridges the physical environment with digital monitoring by creating a connected IoT ecosystem. It consists of four main parts: the Vehicle Sensor Unit (VSU), the connectivity layer, the Firebase Database, and the user interface. The following explains the technical details of every component and the connector which helps them to work together seamlessly.

### Sensors and Devices

The Vehicle Sensor Unit (VSU) is the component that is responsible for the acquisition of data in RoadBounce system [16]. It is a compact module responsible for edge level processing. It includes the following key sensors and devices:

#### Raspberry-Pi 4:

It is the central processing unit of the Vehicle Sensor Unit (VSU). It uses filters and applies machine learning models to the sensor data collected to detect anomalies and transmits processed data via its built-in WiFi back to the central server using mobile hotspot connectivity [16].

#### MPU6050:

MPU6050 is a 6-axis motion sensor that combines 3 axis gyroscope and 3 axis accelerometer. It is the main inertial sensor to determine the physical reaction of the vehicle to the road surface. It reads live information on vibrations, tilts and unexpected car orientation changes, which are vital in detecting road abnormalities [17].

#### NEO-M8N GPS module:

It records the exact latitude/longitude when identifying anomalies. It labels all of them with precise positioning information to enable transmission of geotagged potholes to Firebase by Raspberry Pi. It enables clustering, route warnings and precise mapping of hazards [16].

#### 12v/ 5 amp battery

It is used as the primary power source for the system. It provides portable and reliable power to the Raspberry Pi and other connected components, allowing the project to operate independently without a direct electrical outlet.

#### Step Down Voltage Regulator

The XL4016 Step-Down Voltage Regulator is used to convert the 12V output from the battery into a stable 5V supply required by the Raspberry Pi. It is a high-efficiency DC-DC buck converter capable of handling higher current loads with minimal power loss. The regulator helps protect the Raspberry Pi from voltage fluctuations and ensures smooth and stable system operation.

### Connectivity

This component enables the hardware to communicate with Firebase in real time while the vehicle is moving. For a vehicle in motion, wired connections are not feasible, which is why the following is used:

- Raspberry-Pi 4 Integrated Wi-Fi Module:

The Wi-Fi is inbuilt in the Raspberry Pi. This enables the VSU to be linked to hot spot connection of the mobile phone. When any anomaly is detected, the Raspberry Pi transfers the information and the GPS position locally and transmits the data via its Wi-Fi to the server. That is seen on the map of the mobile app, notifying other drivers too [16].

### Data Processing

From the vehicle, the data is sent to the Firebase for further processing:

- Aggregation of data: Data from multiple vehicles is collected so that road conditions can be tracked in real time across an area.

- Clustering: To prevent overcrowding the map with various entries for the same anomaly, data from different vehicles reporting on the same location is grouped, for instance, different vehicles reporting the same pothole on the same road [16].

### User Interface

The user interface is used by the driver to communicate with the system. The application for RoadBounce provides an engaging, real-time experience:

- The Map: Different Road conditions are represented by color-coded pins on a map where the app displays anomalies that have been identified. For instance, blue pins represent a certain anomaly while red pins represent another one.

- Real-Time Alerts: The app checks for local road irregularities by communicating with the server as the driver moves. The software instantly notifies the driver when an anomaly is spotted ahead, allowing the driver to take the necessary actions (such as slowing down or rerouting).

## Hardware Specifications

Table 3-1 displays a list of all specifications of the hardware devices that were essential for the completion of the project.

Table 3-1: Hardware Specifications

| Hardware | Specifications |
| --- | --- |
| MPU6050: | - Device Type: 6-Axis MEMS IMU (Accelerometer + Gyroscope) - Interface: I²C Communication (400 kHz max) - Operating Voltage: 2.375V – 3.46V - Resolution: 16-bit Analog-to-Digital Converter (ADC) - Digital Motion Processor (DMP): Integrated DMP - Accelerometer Ranges: ±2g, ±4g, ±8g, ±16g - Gyroscope Ranges: ±250°/s, ±500°/s, ±1000°/s, ±2000°/s Temperature Range: -40°C to +85°C |

| Raspberry-Pi 4: | - Processor: Quad-core Cortex-A72 (ARM v8) 64-bit SoC 1.5 GHz - RAM: 2GB, 4GB, or 8GB LPDDR4- 3200 SDRAM - Networking: Gigabit Ethernet - Wireless: 802.11ac Wi-Fi, Bluetooth 5.0 - USB Ports: 2 × USB 3.0, 2 × USB 2.0 - Video Output: 2 × micro-HDMI ports - Storage: microSD card slot for OS and data storage - Power Supply: 5V DC via USB-C connector (minimum 3A) |
| --- | --- |
| Neo-M8N | - Receiver: 72-channel u-blox M8 engine (GPS L1C/A, GLONASS L1OF, BeiDou B1I, Galileo E1B/C, QZSS, SBAS) - Accuracy: 2.5m CEP (GPS), 2.0m CEP (GPS+GLONASS) - Position Update: Up to 10 Hz (10 times/second) - Sensitivity: -167 dBm acquisition, -167 dBm tracking - Time-to-First Fix: Cold start 26s, Hot start <1s - Interface: UART TTL serial (9600 baud default, configurable up to 460800) - Voltage: 1.71V–1.89V (core), 2.7V–3.6V (I/O); module boards typically accept 3.3V–5V - Current: ~67mA continuous (acquisition), ~11mA Power Save Mode - Antenna: Active ceramic patch with built-in SAW filter + LNA (u.FL connector) - Protocols: NMEA 0183 + UBX binary - Operating Temp: -40°C to +85°C - Dimensions: 12.2 × 16.0 × 2.4 mm (baremodule) |
| 12V / 5A Battery | • Type: Rechargeable Lithium-Ion Battery Pack • Output Voltage: 12V DC • Current Capacity: 5A • Typical Capacity: 5000mAh – 10000mAh • Application: Powers Raspberry Pi, sensors, and other connected hardware • Protection Features: Overcharge, over-discharge, and short-circuit protection • Rechargeable: Yes • Operating Temperature: -20°C to +60°C • Output Connector: DC barrel jack / XT60 / screw terminal (depends on model) • Portable and suitable for embedded/IoT systems |
| Step-Down Voltage Regulator | - Device Type: DC-DC Step-Down (Buck) Converter - Input Voltage: 6V – 40V DC - Output Voltage: Adjustable (1.25V – 35V DC) - Output Current: Up to 5A - Conversion Efficiency: Up to 92% - Voltage Regulation: Adjustable using onboard potentiometer - Protection Features: Thermal shutdown and short-circuit protection - Application: Converts 12V battery output to stable 5V for Raspberry Pi and sensors - Cooling: Heat sink support for high-current operation - Common Module: LM2596 / XL4015 based regulator |

Note: Specifications from manufacturer technical documentation [23] [24] [25] [26] [27] [28].

## Communication Protocols

Communication protocols in this system facilitate a smooth flow of road sensor data through hardware acquisition to real-time visualization of data in a mobile environment. The breakdown of the integration of these protocols into the specific components is as given below:

#### Data Acquisition (Sensors to Raspberry Pi 4)

The Raspberry Pi interacts with sensors using low-level serial communication protocols:

- I2C (Inter-Integrated Circuit): It is used for the MPU 6050. It is connected through the SDA (GPIO 2) and the SCL (GPIO 3) pins of the Pi. The Pi queries the MPU 6050 of accelerator and gyroscope data [29].

- UART (Universal Asynchronous Receiver-Transmitter): UART (Universal Asynchronous Receiver-Transmitter): The NEO-M8N GPS module supports UART communication. It can transmit NMEA sentences (text-based location data) over the TX/RX pins to a Raspberry Pi or microcontroller and can also use the UBX binary protocol for more efficient data transfer.

#### Edge Processing & Connectivity

Once the Pi receives the raw data, it processes it using 1D CNN machine learning model to identify anomalies [30].

- Wi-Fi (802.11ac): Raspberry Pi 4 relies on the inbuilt Wi-Fi to access mobile hotspot.

- DHCP: Gives the Pi an IP address which is automatically assigned by the hotspot, and it can be used to access the internet.

#### Data Transmission (Pi to Firebase Server)

To move data from the edge (Pi) to the cloud, the system utilizes application-layer protocols:

- HTTPS (Hypertext Transfer Protocol Secure): The Raspberry Pi transmits the anomaly data that is processed to the Firebase Realtime Database [30].

- JSON (JavaScript Object Notation): It is the default format of these transmissions that the information is organized in pairs of key-values.

#### Real-time Visualization (Firebase to Phone App)

The result is displayed on the map of the mobile application using a continuous sync protocol:

- WebSockets: Firebase relies on WebSockets to provide open and two-way communication between the phone app and the server [30].

- Real-time Synchronization: Through Pi, the server receives the data sent by it straight to Firebase which is updated on the phone app without any refresh of the app [30].

## Data Modeling

In this project, how sensor data, anomaly information, and user details will be stored, structured, and managed in the system is defined by data modeling. It makes sure that database is designed based on the requirements of the system.

Because the system will be dealing with constant car information, real-time anomaly detection, updates based on GPS, Firebase Realtime Database will be extremely efficient in terms of making sure that data will be synchronized and scaled. It is also chosen due to its simplicity and ease of use.

### NoSQL Database

Data is organized into collections. Unlike traditional tables, each collection contains documents stored in JSON format. This allows for a flexible schema where data is represented as key-value pairs, enabling the nesting of complex information directly within a single record.

Figure 3-1 illustrates the Firebase database structure used in the RoadBounce system. For all user related information. User-related information, alerts, current trip details, route history, and location data are maintained within nested document structures, enabling efficient storage and retrieval of real-time road anomaly information.

Figure 3-1: User Collection

Figure 3-2 illustrates the Firebase document structure used to store Vehicle Sensor Unit (VSU) information. The document contains sensor status, calibration details, operational state, and the latest event data, including timestamp and GPS location.

Figure 3-2: VSU Collection

Figure 3-3 illustrates the Firebase collection used to store detected road anomalies. Each document contains information such as anomaly type, location coordinates, severity level, detection timestamp, verification status, and the associated Vehicle Sensor Unit (VSU).

Figure 3-0-3: Road_Anomaly Collection

Figure 3-4 presents the map cluster collection used to group nearby anomaly reports into clusters. Each cluster stores its location, associated anomalies, contributing VSUs, and verification status to reduce duplicate reports and improve data reliability.

Figure 3-4: Map_Clusters Collection

## Workflow Diagram

The workflow diagram of the RoadBounce project is presented below in Figure 3-5, and indicates the interdependence between the hardware sensors, the mobile application and the Google Maps API to provide real-time road conditions information.

Figure 3-5: Workflow Diagram

## Third-Parties Dependencies

The third-party dependencies are divided into hardware, software, and external services. Below is the categorized list:

Based on the RoadBounce project report, third-party dependencies are categorized into hardware, software, and external services. Below is the categorized list:

### Hardware Dependencies

These are the physical electronic components integrated into the Vehicle Sensor Unit (VSU):

- MPU-6050: A 6-axis motion sensor that combines a 3-axis accelerometer and a 3-axis gyroscope, used to detect vibrations and vehicle orientation changes.

- NEO-M8N GPS Module: GPS modules that provide real-time location, speed, and geotags for detected road anomalies.

- Raspberry Pi 4: The central processing unit for the VSU, responsible for data filtering, local storage, and transmission via built-in Wi-Fi.

### Software Frameworks & Tools

These are the development environments and frameworks used to build and manage the system's software:

- Kotlin: A modern programming language used for developing the Android mobile application.

- XML: A markup language used for designing and structuring the Android app user interface.

- Python: A flexible language used for implementing artificial intelligence and machine learning models within the system.

- Android Studio: The official Integrated Development Environment (IDE) for mobile app development.

- Google Colab: A cloud-based platform used for running and testing machine learning and data analysis code.

### External Services & Cloud Infrastructure

These are third-party platforms that provide critical data storage and visualization services:

- Firebase: Used by the Raspberry Pi to store and transmit geotagged pothole data to the central system.

- Google Maps: The application uses map displays to visualize road conditions, mark anomalies, and provide routing based on real-time data. Although not explicitly listed in the "Software" section, it is essential for the system's functionality.

# Chapter 4

# Software Development

## Implementation Overview

The implementation phase focused on translating the Client-Server and Pipe-Filter architectures defined in the system design into executable code. The development was split into two parallel workstreams, as outlined in the project timeline:

- Vehicle Sensor Unit (VSU) Development: Involving Python programming on the Raspberry Pi 4 to handle sensor acquisition, signal processing, and data transmission.

- Mobile Application Development: Using Android Studio and Kotlin, Xml to build the user interface and map visualizations.

## VSU Software Implementation (Python & Edge Processing)

The core logic for the Vehicle Sensor Unit was implemented using Python due to its extensive library support for hardware interfacing and machine learning

### Sensor Interface Layer

The implementation began with establishing low-level communication with the hardware components:

- MPU-6050 Interfacing: The smbus library was utilized to communicate with the MPU-6050 via the I2C protocol. A Python class MPU6050_Handler was created to initialize the sensor, wake it from sleep mode, and read raw 16-bit integers from the accelerometer and gyroscope registers.

- GPS Parsing: The NEO-6M GPS module was accessed via the UART serial port. A background thread was implemented to continuously read the serial stream. The software parses standard NMEA sentences to extract latitude, longitude, and speed in real-time.

### Signal Processing and Anomaly Detection

To implement the Pipe-Filter architecture, the data flow was executed as a series of sequential processing steps:

- Calibration: A startup routine was coded to capture the first 100 readings while the vehicle is stationary. The average of these readings is calculated as an offset (bias) and subtracted from subsequent readings to zero the sensors (FR3).

- Noise Filtering: A digital Bandpass Filter was implemented using the scipy.signal library. This filter removes low-frequency noise (gravity/engine idling) and high- frequency sensor jitter, isolating the specific frequency range associated with road impacts (FR5).

- ML Inference: The trained 1D CNN model was loaded onto the Raspberry Pi using TensorFlow Lite for faster inference. The buffered sensor data is passed to this model, which outputs a probability score labelling the road segment as an anomaly.

## Mobile Application Implementation

The mobile application serves as the visual front-end and is implemented using Android Studio, with Kotlin and XML used for application development and user interface design.

### User Interface and State Management

The UI follows the prototypes designed in Chapter 2.

- State Management: The application uses the Provider package to manage the state of the user's location and the list of fetched anomalies. This ensures that when new data arrives from the server, the UI rebuilds efficiently without freezing.

- Navigation: The routing logic uses Google Maps API. When a user searches for a destination, the app requests a route polyline. A custom algorithm then checks if any stored anomalies in the database intersect with the route coordinates to generate the "Safe" vs. "Fast" route options.

### Real-Time Map Visualization

The integration with Google Maps was achieved using its api.

- Markers: Anomalies are rendered as custom Marker objects. The logic dynamically assigns a red icon for potholes and a blue icon for speed bumps.red, yellow or green icons based on the possible impact that could result from the vehicle’s impact with the anomaly.

- Live Updates: A StreamBuilder was connected to the Firebase database. This utilizes WebSockets to listen for changes. As soon as a VSU detects a pothole, the database updates, and the marker appears on the driver’s map instantly.

## Problems Encountered and Solutions

### Problem 1: Real-Time Data Collection from MPU6050 and GPS

In real-time deployment, the Raspberry Pi receives data from two different hardware modules: the MPU6050 sensor and the GPS module. The MPU6050 provides acceleration and gyroscope readings, while the GPS module provides latitude, longitude, and speed. A key challenge was combining both sources so that every anomaly detection event contains both motion features and location information.

#### Solution: Real-Time Sensor and GPS Fusion

The system continuously reads acceleration and gyroscope values from the MPU6050 at 5Hz. At the same time, the GPS module runs in a background thread and keeps the latest available GPS fix. Each sensor sample is then combined with the latest GPS latitude, longitude, and speed, shown in Figure 4-1.

Figure 4-1: Combining MPU6050 and GPS Data

This ensures that each detected pothole event contains the correct GPS location and vehicle speed.

### Problem 2: Signal Noise and Vehicle Tilt

Accelerometers measure both vehicle motion and gravity. If the sensor is mounted at an angle, gravity affects the Z-axis reading and can create false anomaly signals. In addition, road texture and engine vibration can introduce noise into the acceleration signal.

#### Solution: Physics-Based Signal Preprocessing

The system estimates the tilt of the sensor and removes the gravity component from the vertical acceleration. Then, a bandpass filter is applied to isolate pothole-related vibration patterns, as shown in Figure 4-2 below.

Figure 4-2: Tilt Correction and Filtering

This produces a cleaner vertical acceleration signal that is more suitable for pothole detection.

### Problem 3: False Positives in Anomaly Detection

Simple acceleration thresholds can mistake braking, turning, or road vibration for potholes. A more reliable method was needed to distinguish anomalies from normal driving behaviour.

#### Solution: CNN-Based Detection

Instead of using only fixed acceleration thresholds, the system uses a trained CNN model. The Raspberry Pi creates the same feature window used during training and passes it to the CNN, which can be seen below in Figure 4-3.

Figure 4-3: CNN Input Preparation and Prediction

The CNN returns a pothole probability. An event is generated only when the probability is greater than the selected threshold.

### Problem 4: Repeated Detections for the Same Pothole

A single pothole can appear in several consecutive sensor windows. Without filtering, the system may send multiple alerts for the same pothole.

#### Solution: Cooldown-Based Duplicate Suppression

After a pothole is detected, the system waits for a cooldown period before allowing another event. The logic for cooldown is represented by Figure 4-4.

Figure 4-4: Cooldown Logic

This prevents repeated Firebase alerts for the same road anomaly.

### Problem 5: Low-Speed Noise and False Positives

When the vehicle is stopped or moving slowly, the MPU6050 can still detect engine vibration or small body movements. These should not be reported as potholes.

#### Solution: GPS-Based Speed Threshold

The system checks the average GPS speed over the latest window. If the vehicle speed is below the selected threshold, the window is ignored. As shown in Figure 4-5, the calculated average speed is compared against a predefined threshold to filter out stationary or slow-moving vehicle data.

Figure 4-5: Speed Threshold Check

This reduces false positives when the vehicle is stationary or moving very slowly.

### Problem 6: Severity and Impact Interpretation

Detecting a pothole is useful, but the system should also describe how serious the impact was. A small bump and a dangerous pothole should not be treated equally.

#### Solution: Severity and Impact Scoring

After a pothole is detected, the system calculates severity using the maximum filtered vertical acceleration. The severity is then converted into a label and impact description. Figure 4-6 illustrates the severity and impact calculation mechanism used to categorize detected road anomalies.

Figure 4-6: Severity and Impact Calculation

The final detection event includes the probability, severity, impact, GPS location, and vehicle speed. As shown in Figure 4=7, the event record includes the anomaly probability, severity score, impact description, GPS coordinates, and vehicle speed before being uploaded to Firebase.

Figure 4-7: Anomaly Details

## Coding Standards

To ensure code maintainability, readability, and seamless collaboration between team members, the RoadBounce project strictly adhered to established coding standards for both the Python (VSU) and Java (Mobile App) environments. The development environments (Google Colab and Android Studio) were configured to enforce these rules automatically.

### Indentation

Indentation is critical for defining the scope and hierarchy of the code. Specific indentation rules were adopted based on the language:

#### Python (VSU Module):

- PEP 8 style guide was followed, using 4 spaces per indentation level.

- Tabs were strictly prohibited to avoid inconsistencies across different operating systems (Raspberry Pi Linux vs. Windows Dev machines).

- Continuation lines were indented vertically to align with the opening delimiter (e.g., inside parentheses) to improve readability.

### Declaration

Variable and function declarations were structured to maximize clarity and type safety.

#### Variables:

- Strong typing (int, String, double) was utilized wherever possible to catch errors at compile time. For values that do not change, the final or const keywords were used to optimize performance.

- In Python, while dynamically typed, Type Hints were used in function signatures to make the data flow explicit and understandable.

#### Classes and Imports:

- All imports were declared at the absolute top of the file.

- Unused imports were removed during the refactoring phase.

- Class fields were declared at the top of the class body, followed by constructors, and then methods.

### Naming Convention

Consistent naming conventions were applied to distinguish between variables, classes, and constants. Table 4-1 displays a list of all Naming Conventions that were  used in the coding of Scripts for hardware and Mobile Application.

Table 4-1: Naming Convention Table

| Element | Python Standard (VSU) | Mobile App Standard | Example (RoadBounce) |
| --- | --- | --- | --- |
| Variables | snake_case | lowerCamelCas e | accel_z (Py), userLocation |
| Functions | snake_case | lowerCamelCas e | detect_pothole() (Py), fetchRoute() |
| Classes | PascalCase | PascalCase | SensorHandler (Py), MapScreen |
| Constant s | UPPER_SNAKE_CAS E | lowerCamelCas e (kPrefix) | GRAVITY_CONS T (Py), kMapZoom |
| File Names | snake_case.py | snake_case.kt | data_processing.py, home_screen.kt |

### Statement Standard

Statement standards defined the structure of individual lines of code and blocks.

#### Line termination:

- Kotlin: Statements are terminated by newline.

- Python: Statements are terminated by a newline. Semicolons were strictly avoided.

#### Line Length:

- A maximum line length of 80 characters for Kotlin and 79 characters for Python was maintained. This ensures that code files can be viewed side-by- side on a standard monitor without horizontal scrolling. Long statements were wrapped using parentheses.

#### Block Structure:

- Kotlin: Braces {} were used for all control flow statements (if, else, for), even for single-line blocks, to prevent ambiguity.

- Python: Colons: were used to define blocks, relying on strict indentation to define the scope.

#### Comments:

- Inline comments were used sparingly for complex logic.

- Docstrings (Triple quotes “") were used in Python to describe the purpose, arguments, and return values of complex functions.

## Development Environment

This section describes the development environment, tools, technologies, and third-party libraries used in the implementation of the RoadBounce system. The selection of each tool was based on its compatibility with sensor-based data processing, machine learning support, ease of deployment, and scalability.

### Programming Language Selection

The primary programming language used for the development of this project is Python.

Reasons for using Python:

- Extensive support for machine learning and deep learning

- Availability of mature libraries for signal processing

- Easy integration with hardware platforms such as Raspberry Pi

- Rapid development and debugging capabilities Python was used for:

- Sensor data preprocessing

- Signal filtering

- Feature extraction

- Training and evaluation of the 1D CNN model

### Development Platform

The development environment was deployed using:

- Google Colab for initial model training and experimentation

- Local Python Environment for development and testing

- Raspberry Pi OS (Linux-based) for deployment

Reasons:

- Google Colab provides free GPU/CPU resources, useful for faster CNN training

- Local environment allows fine-grained control over libraries and versions

- Raspberry Pi OS ensures compatibility with embedded deployment

### Data Processing and Scientific Libraries

Table 4-2 displays a list for all the third-party Python libraries that were used for data handling and preprocessing:

Table 4-2: Third-party python libraries

| Library | Purpose |
| --- | --- |
| NumPy | Numerical operations on sensor data |
| Pandas | Reading and managing CSV sensor files |
| SciPy | Butterworth band-pass filtering |
| Matplotlib | Signal visualization and debugging |

### Machine Learning and Deep Learning Frameworks

The TensorFlow and Keras frameworks were used to develop and train the 1D CNN model.

#### Reasons for using TensorFlow/Keras:

      - Native support for 1D convolution layers

      - Easy model definition using high-level APIs

      - Built-in optimization tools

      - Support for TensorFlow Lite, which is essential for embedded deployment

The trained CNN model was converted to TensorFlow Lite (.tflite) format to reduce computational overhead on the Raspberry Pi.

### Signal Processing Tools

To improve signal quality, SciPy’s signal processing module was used to implement:

      - Butterworth band-pass filter

      - Frequency-domain noise suppression

This allowed the system to isolate meaningful vibration patterns caused by road anomalies while eliminating unwanted noise.

### Mobile Application Development Environment

The mobile application was developed using Kotlin and XML in Android Studio.

#### Reasons for using Kotlin and XML:

- Native Android development: Kotlin and XML are officially supported for Android application development.

- Better hardware and platform integration: Android Studio provides direct access to Android features such as GPS, internet permissions, background services, and notifications.

- Reliable performance: Kotlin-based Android applications run efficiently on Android devices and are suitable for real-time data display.

- Structured UI design: XML layouts allow clear separation between interface design and application logic.

- Firebase integration: Android Studio provides strong support for Firebase services such as Realtime Database, Firestore, and authentication.

The Android application communicates with the Firebase to fetch and display anomaly detection data in real time.

### Hardware Development Environment

The system was deployed on a Raspberry Pi, chosen due to:

- Sufficient processing power for AI inference

- Compatibility with Python and TensorFlow Lite

- Support for GPS and GSM modules

- Linux-based OS for server hosting

The Raspberry Pi acted as:

      - Sensor data acquisition unit

      - Local inference device

      - Data transmission gateway

The selected development environment ensured smooth integration of hardware, AI models, and mobile applications.

The combination of Python-based tools, TensorFlow, Raspberry Pi, Android Studio enabled efficient development, deployment, and scalability of the RoadBounce system.

## Software Description

Following are the major modules that were produced for the RoadBounce project, illustrated by a Class diagram as shown in Figure 4-8:

- Sign-In process

- User Dashboard

- Route Selection

- User History

- Anomaly Info

- Data Filtering

- Anomaly Detection

- Anomaly Recording with GPS coordinates

- Environmental Adaptation

- Anomaly Marking

Figure 4-8: Class Diagram

### Sign-In Process

Firebase Authentication is used to verify the user's email and password credentials during the sign-in process. As shown in Figure 4-9, the application authenticates users using the signInWithEmailAndPassword() method and performs an additional email verification check before granting access to the system.

Figure 4-9: Sign-In Code

After successful authentication, the application checks whether the user's email address has been verified. If the email has not been verified, the user is signed out and access is denied. Once the verification is confirmed, the user's UID and profile information are stored in a UserModel, which is later used throughout the application session.

The secure login functionality was validated through the test case shown in Table 4-3 to ensure that only authenticated users can access the mobile application.

Table 4-3: Secure Login for Mobile App

| Field | Description |
| --- | --- |
| Test ID | E3-US1-TC1 |
| US ID | E3-US1 |
| Test Case Description | Verify that only authenticated users can access the mobile app. |
| Inputs | Valid email/password and invalid email/password |
| Expected Result | Valid credentials allow login; invalid credentials show an error message. |
| Actual Result | Valid users are authenticated successfully. Invalid users receive an authentication error. |
| Status | Pass |

### User Dashboard

The dashboard gets the road anomaly data from Firebase Realtime Database and changes the user interface when new anomalies are detected. A real-time listener is registered as demonstrated in Figure 4-10 to keep watching changes to the database and to automatically update the information about the anomalies displayed.

Figure 4-10: Dashboard Code

The dashboard is listening to Firebase Realtime Database for road anomaly updates. When the anomalies data changes, the listener updates the anomalies list, and the Compose UI and map markers are refreshed automatically. This will enable the dashboard to show the current road conditions without having to reopen the screen.

To verify the road anomalies detected are correctly visualized and updated in real time, the map visualization functionality was tested using a test case as shown in Table 4-4.

Table 4-4: Display Road Conditions on Map

| Field | Description |
| --- | --- |
| Test ID | E3-US2-TC1 |
| US ID | E3-US2 |
| Test Case Description | Verify that road anomalies are displayed on the map in real time. |
| Inputs | Firebase anomaly records containing latitude, longitude, severity, and type |
| Expected Result | The map displays anomaly markers at the correct GPS coordinates. |
| Actual Result | Markers are shown accurately and update when Firebase data changes. |
| Status | Pass |

### Route Selection

The route recommendation module analyses the available routes with the real-time road anomaly information acquired from the system. Each route is assessed by distance, journey time and the severity of the anomalies, as illustrated in Figure 4-11, to identify which is the safest route to take.

Figure 4-11: Route Selection Code

The route selection module is a comparison between available routes based on real-time anomaly data. All routes are scanned for anomalies in the immediate vicinity, safety penalties are assigned, distances, times and the severity of the anomalies are used for scoring. The safest route will have the lowest composite score and will be shown as the recommended route.

The test case presented in Table 4-5 was used to validate the route-recommendation functionality to ensure that the route options are arranged in the correct order according to the real-time information of the road conditions.

Table 4-5: Best Route Options Based on Real-Time Data

| Field | Description |
| --- | --- |
| Test ID | E3-US4-TC1 |
| US ID | E3-US4 |
| Test Case Description | Verify that route options are ranked using real-time anomaly data. |
| Inputs | Start location, destination, route polylines, Firebase anomaly list |
| Expected Result | Safer routes with fewer or lower-severity anomalies are preferred. |
| Actual Result | Routes are scored and sorted based on anomaly penalty, duration, and distance. |
| Status | Pass |

### User History

The trip history module retrieves previously completed trips from Firebase and displays them to the user for future reference. As shown in Figure 4-12, the application listens for updates to the user's trip records and dynamically displays the retrieved trips in a scrollable list.

Figure 4-12: User History Code

The history screen listens to the user's trip records from Firebase. If trip records exist, they are displayed in a scrollable list using a LazyColumn. If no trips are available, the application displays an empty-state message indicating that no trip history has been recorded.

The trip history functionality was validated using the test case presented in Table 4-6 to ensure that completed trips are correctly retrieved and displayed in the application's history section.

Table 4-6: List of History of Trips

| Field | Description |
| --- | --- |
| Test ID | E3-US6-TC1 |
| US ID | E3-US6 |
| Test Case Description | Verify that completed trips are displayed in the history section. |
| Inputs | Logged-in user and saved trip records |
| Expected Result | Trip history shows date, route, distance, and trip details. |
| Actual Result | Trips are displayed in a scrollable list; empty state appears when no trips exist. |
| Status | Pass |

### Anomaly Info Card

Anomaly information card displays more information about the detected road anomalies. The application translates the severity of the anomaly into easy-to-understand labels, messages for users to act upon and be alerted to the potential risk, as illustrated in Figure 4-13.

Figure 4-13: Anomaly Info Card Code

If the user clicks on an anomaly marker, the application will present an information card with a message indicating the severity of the anomaly and an advisory message. The road anomaly information displayed assists the user to determine the level of risk of the detected road anomaly and to make safer driving decisions.

Functionality of the anomaly information card was tested using the test case shown in Table 4-7 to facilitate the presentation of the anomaly information and advisory information to the users.

Table 4-7: Anomaly Info Card

| Field | Description |
| --- | --- |
| Test ID | E3-US7-TC1 |
| US ID | E3-US7 |
| Test Case Description | Verify that tapping an anomaly marker opens an information card. |
| Inputs | Anomaly marker with severity, probability, and GPS coordinates |
| Expected Result | An info card displays anomaly severity and user guidance. |
| Actual Result | The anomaly card appears and can be dismissed by the user. |
| Status | Pass |

### Data Filtering

Raspberry Pi signal filtering is used to enhance the reliability of detection before sending the anomaly data to Firebase. The mobile app only receives the pre-processed anomaly information, which include the filtered detection results and the anomaly information, through a HazardModel, as illustrated in Figure 4-14.

Figure 4-14: Filtered Data Consumption Code

Before the anomaly data is uploaded to Firebase, the signal is filtered on the Raspberry Pi in the deployed system. The Android application takes pre-processed anomaly records and visualizes them with HazardModel. It helps to keep the mobile application's weight to a minimum while keeping the user informed about filtered detection results.

To ensure that only processed anomaly records are received and displayed in the mobile application, the function to handle data filtering was tested with the test case given below in Table 4-8.

Table 4-8: Data Filtering Accuracy

| Field | Description |
| --- | --- |
| Test ID | E3-US8-TC1 |
| US ID | E3-US8 |
| Test Case Description | Verify that the mobile app receives processed anomaly data after filtering. |
| Inputs | Filtered anomaly record from Firebase |
| Expected Result | The app displays only processed anomaly records, not raw noisy sensor data. |
| Actual Result | Filtered anomaly records are received and parsed correctly. |
| Status | Pass |

### Anomaly Detection

The trained CNN model is employed at the Raspberry Pi to detect the road anomalies. After an anomaly is detected, the outcome is uploaded to Firebase Realtime Database. The Android application registers a Firebase listener to receive the detected anomaly events and update real-time the dashboard as shown in Figure 4-15.

Figure 4-15: Anomaly Detection Data Listener Code

The “anomaly detection” is done by the hardware/CNN pipeline. The Android application subscribes to Firebase in response to anomaly events being detected. Each new anomaly added to Firebase is accepted by the application and thus automatically updates the dashboard.

The anomaly reporting functionality was tested with the test case shown in Table 4-9 to confirm that anomalies detected on the road are reported in the mobile application as soon as they are detected.

Table 4-9: Real-Time Road Anomaly Detection

| Field | Description |
| --- | --- |
| Test ID | E2-US3-TC1 |
| US ID | E2-US3 |
| Test Case Description | Verify that detected road anomalies appear in the app in real time. |
| Inputs | Firebase anomaly event produced by the hardware detector |
| Expected Result | The anomaly appears on the map without restarting the app. |
| Actual Result | Firebase listener updates the app immediately when anomaly data changes. |
| Status | Pass |

### Anomaly Recording with GPS Coordinates

Every detected road anomaly needs to have a valid GPS location in order to map the location. The application fetches latitude and longitude data from records in Firebase, checks them, and then shows anomalies on the map as shown in Figure 4-16.

Figure 4-16: GPS Coordinate Parsing Code

Anomaly records should have valid GPS coordinates. The repository securely validates latitude/longitude fields from Firebase and discards any bad data. This will ensure that the map doesn't show an incomplete or corrupted anomaly data.

The anomaly recording function, which uses GPS, was checked in the test case given in Table 4-10 to validate whether the anomaly record includes valid location information before presenting it to users.

Table 4-10: Road Anomaly Recording and Storage with GPS Coordinates

| Field | Description |
| --- | --- |
| Test ID | E2-US6-TC1 |
| US ID | E2-US6 |
| Test Case Description | Verify that anomalies include GPS coordinates before display. |
| Inputs | Firebase anomaly record with latitude and longitude |
| Expected Result | Valid GPS records are displayed; invalid records are skipped. |
| Actual Result | Coordinates are parsed correctly and used for map markers. |
| Status | Pass |

### Environmental Adaptation

Environmental conditions and anomaly severity influence the route recommendation process. As shown in Figure 4-17, the application assigns different penalty values to detected anomalies based on their severity level, ensuring that routes containing more severe road hazards are ranked lower during route selection.

Figure 4-17: Severity-Based Route Penalty Code

The hardware preprocessing and detection pipeline is mainly responsible for environmental adaptation. The Android version of the app adjusts routing recommendations based on anomaly severity. Routes with bad road conditions will be less likely to be recommended, as will be the case for high-severity anomalies.

The functionality of false alert filtering was validated through the test case presented in Table 4-11 ensuring that environmental effects and the severity of anomalies are dealt with correctly when evaluating and recommending routes.

Table 4-11: False Alert Filtering for Environmental Conditions

| Field | Description |
| --- | --- |
| Test ID | E2-US5-TC1 |
| US ID | E2-US5 |
| Test Case Description | Verify that severity-aware route scoring reduces unsafe route recommendations. |
| Inputs | Routes containing low, medium, and high severity anomalies |
| Expected Result | Routes with high-severity anomalies receive higher penalties. |
| Actual Result | High-severity anomalies are weighted more heavily in route scoring. |
| Status | Pass |

### Anomaly Marking

Detected road anomalies are visualized on Google Maps to help users identify hazardous road conditions. As shown in Figure 4-18, anomaly markers are placed using GPS coordinates and customized according to anomaly severity to improve visibility and user awareness.

Figure 4-18: Anomaly Marker Code

The application places road anomalies on Google Maps using their GPS coordinates. Marker appearance is determined by anomaly severity, allowing users to quickly identify and differentiate hazardous road areas.

The map marking functionality was validated using the test case presented in Table 4-12 to ensure that detected road anomalies are displayed at the correct GPS locations and represented accurately on the map.

Table 4-12: Marking of Road Anomalies with GPS Coordinates

| Field | Description |
| --- | --- |
| Test ID | E2-US7-TC1 |
| US ID | E2-US7 |
| Test Case Description | Verify that detected anomalies are marked on the map using GPS coordinates. |
| Inputs | Anomaly records with latitude, longitude, type, and severity |
| Expected Result | Each anomaly appears as a marker at the correct location. |
| Actual Result | Markers are displayed correctly and update with Firebase data. |
| Status | Pass |

# Chapter 5

# Software Deployment

The process of making a developed application available for end users to access and use in a real-world environment is called software deployment. This project involves the deployment of software in two phases: the embedded software running on the Raspberry Pi 4 hardware platform, and the mobile software application on the Google Play Store on Android devices. The mobile application is the interface that will be used by the user and displays the anomalies detected by the road detection system as coloured markers on the interactive map in real time. By using this application, the user can view the event data sent from the Raspberry Pi via the Firebase Realtime Database, without any direct interaction with the device. By publishing it in the Google Play Store, it can be made available to a large audience of Android users, updates are distributed automatically and there is a trusted installation channel. This chapter details the deployment configuration, environment requirements, installation procedures and guidelines for the hardware and mobile application elements of the system [31].

## Deployment Process Description

There were two parts of this project

- Software deployment on Raspberry Pi

- Deployment of application on App store

The software deployment involves making each of the software components operational in the target environment, such as the CNN-based detection script that runs on the raspberry pi onboard and the mobile application that end users can use. The two phases combined make sure that the full RoadBounce system is completely functional and available for actual application in the field.

### Raspberry Pi Deployment

The road anomaly detection module was deployed directly on a Raspberry Pi as a standalone Python service. This component is responsible for reading real-time accelerometer data from the MPU6050 sensor, running inference using the trained CNN model, and publishing detected pothole events to Firebase. Three files are required for this deployment: the trained model (public_road_anomaly_cnn.h5), and two NumPy normalisation arrays (public_cnn_norm_mean.npy and public_cnn_norm_std.npy).

#### Files Transferred to Raspberry Pi

The following files were transferred to the Raspberry Pi:

Table 5-1: Files transferred to Raspberry Pi

| File | Purpose |
| --- | --- |
| pi_pothole_detector.py | Main detection script |
| public_road_anomaly_cnn.h5 | Trained CNN model weights |
| public_cnn_norm_mean.npy | Per-feature mean used during training normalisation |
| public_cnn_norm_std.npy | Per-feature standard deviation for normalisation |
| firebase_credentials.json | Firebase service account key for publishing events |

#### Environment Setup

The necessary Python packages were installed on the Raspberry Pi using Pip. The .h5 Keras model was directly loaded with TensorFlow. There is also a converted .tflite for tflite_runtime which can be used for resource constrained deployments to reduce memory consumption and increase inference speed on the Pi.

#### Running of Detector

Using the following commands, the script is launched from the terminal:

python3 pi_pothole_detector.py \

--model public_road_anomaly_cnn.h5 \

--mean public_cnn_norm_mean.npy \

--std public_cnn_norm_std.npy \

--source mpu6050 \

--threshold 0.75 \

--firebase-credentials firebase_credentials.json \

--firebase-collection road_anomalies \

--device-id pi-road-node-001

#### Firebase Publishing

The detected events get published to Firebase on Firestore or Realtime Database, depending on the configuration flags. All published event documents contain:

- The GPS device ID, GPS time, GPS location, and GPS speed

- A CNN probability score and threshold were employed

- Severity score, severity label, and impact description

- Raw accelerometer detections at the point of detection

In the absence of Firebase credentials, events are logged locally to the terminal, which can be very useful in development and testing without a live Firebase project.

## System Requirements for Application

Before installing the RoadBounce application, the following requirements shown in Table 5-1 must be met:

Table 5-2: System Requirements

| Component | Requirement |
| --- | --- |
| Operating System | Android 8.0 (Oreo) or above |
| Internet Connection | Required for Firebase synchronization |
| GPS | Required for location tracking |
| RAM | Minimum 2 GB |
| Storage | Minimum 100 MB free space |
| Google Play Services | Installed and enabled |

### Google Play Console Deployment

The application can be deployed through the Google Play Store using the following procedures:

#### Step 1: Login to Google Play Console

Open https://play.google.com/console and sign in with a Google Developer account as shown below in Figure 5-1.

Figure 5-1: Google Play Console Dashboard

#### Step 2: Create New Application

Figure 5-3 shows below how a new application can be created.

- Click Create App.

- Enter application name RoadBounce.

- Select default language.

- Choose App type.

- Accept Google Play policies.

Figure 5-2: Create Application Page

#### Step 3: Upload Android App Bundle

Figure 5-4 shows the steps that are needed in order to upload an android bundle.

- Navigate to Production Release.

- Click Create New Release.

- Upload the generated app-release.aab.

- Enter release notes.

Figure 5-3: Uploading Android App Bundle

#### Step 5: Configure App Content

In order to fulfill the Google Play publishing requirements, the following application information and policy details must be completed before submitting the application for review:

- Privacy Policy

- Data Safety Form

- Content Rating

- Target Audience

A Privacy Policy page was created using Google Sites and linked with the RoadBounce application in the Google Play Console. The policy explains the use of GPS location, sensor-based road anomaly data, Firebase Realtime Database storage, and the purpose of data collection. This step was completed to fulfill Google Play Store deployment requirements and to inform users about how their data is handled.

As shown in Figure 5-4, the Privacy Policy was created and published using Google Sites, making it publicly accessible through a web URL for users and Google Play review.

Figure 5-4: Privacy Policy Publication Using Google Sites

After publishing the Privacy Policy, the generated URL was added to the Google Play Console, as shown in Figure 5-5. This allows users to access the privacy policy directly from the application's store listing.

Figure 5-5: Privacy Policy Configuration

Following the Privacy Policy configuration, the Data Safety form was completed to declare the application's data collection and sharing practices. As shown in Figure 5-6, the RoadBounce application does not collect user data or share data with third parties.

Figure 5-6: Data Safety Form Configuration

After completing the required policy declarations, the application store listing was configured in the Google Play Console. As shown in Figure 5-7, promotional information and store presentation settings were prepared for the RoadBounce application before submission.

Figure 5-7: Store Listing Configuration

After completing all required configurations, the application release was prepared for review and submission. As shown in Figure 5-8, Google Play Console directs the developer to the Publishing Overview section, where all pending changes are reviewed and submitted to Google for approval.

Figure 5-8: Publishing Overview and Release Submission

#### Step 6: Submit Release for Review

After completing the application configuration and testing setup, the release must be submitted to Google Play Console to review using the following procedure:

- Review all application information and deployment settings.

- Resolve any warnings and policy-related issues.

- Create a closed testing release and upload the Android App Bundle (AAB).

- Submit the release to Google Play Console for review.

- Google reviews the application, store listing, privacy policy, and data safety declarations before approving the release for testing.

#### Step 7: Closed Testing and Production Release

After approval, the application enters the Closed Testing track where selected testers evaluate the system functionality. Figure 5-9 shows the closed testing release status of the RoadBounce application in Google Play Console. All pending changes and release information were reviewed through the Publishing Overview page before submission, as illustrated in Figure 5-10. Once submitted, the application entered the review process, as shown in Figure 5-11. After testing requirements are satisfied and Google grants production access, the application can be published to the Production track and made available to users through the Google Play Store.

Figure 5-9: Closed Testing Release Status in Google Play Console

..

Figure 5-10: Publishing Overview for Release Submission

Figure 5-11: Application Under Review in Closed Testing Track

## Deployment Challenges

During deployment, several challenges were encountered:

- Google Play privacy policy requirements.

- Data Safety declaration configuration.

- Account deletion URL requirement.

- Photo permission declaration.

- Advertising ID declaration.

- Version code conflicts during release creation.

These issues were resolved by updating the application configuration and providing the required policy information before submission.

# References

[1] Y. Alqasi, A. M. A. Jahan, A. A. M. A. Y. Ahmed, "Intelligent Infrastructure for Urban Transportation: The Role of Artificial Intelligence in Predictive Maintenance," Brilliance Research of Artificial Intelligence, vol. 4, no. 2, pp. 625–637, 2024.

[2] CyberSwift, "Road Condition Monitoring System (RCMS): Enhancing Efficiency with AI-Powered Solutions," 17 December 2024. [Online]. Available: https://www.cyberswift.com/blog/road-condition-monitoring-system-rcms-enhancing-efficiency-with-ai-powered-solutions/ . [Accessed: 4 October 2025].

[3] G. Delnevo, N. Bergamaschi, S. Manzoni, P. Salomoni, and C. Prandi, "A Mobile Application for the Crowdsensing of Road Pavements Conditions," in Proc. IEEE Consumer Communications & Networking Conference (CCNC), Las Vegas, NV, USA, 2025.

[4] A. Jahan, S. I. Ahmed, I. A. Malik, K. M. Hassan, M. Saleem, and M. Z. Shafiq, "RoadSense: A Framework for Road Condition Monitoring using Sensors and Machine Learning," IEEE Transactions on Intelligent Vehicles, pp. 1–12, 2024.

[5] C. Stellwagen, P. Fuchs, and C. Sommer, "Hybrid Vehicle-to-X Communication Network by Using LTE-V2X and ITS-G5," IEEE Transactions on Intelligent Transportation Systems, vol. 24, pp. 2638–2650, 2023.

[6] J. Eriksson, L. Girod, B. Hull, R. Newton, S. Madden, and H. Balakrishnan, "The Pothole Patrol: Using a Mobile Sensor Network for Road Surface Monitoring," in Proc. 6th Int. Conf. Mobile Systems, Applications, and Services (MobiSys), Breckenridge, CO, USA, 2008.

[7] M. A. Kamal, "Real Time Road Monitoring System," 2022. [Online]. Available: https://www.researchgate.net/publication/359139782_Real_Time_Road_Monitoring_System . [Accessed: 4 October 2025].

[8] MarketsandMarkets, "Smart Transportation Market Size, Share & Trends," 2024. [Online]. Available: https://www.marketsandmarkets.com/Market-Reports/smart-transportation-market-692.html . [Accessed: 4 October 2025].

[9] Phidgets, "Accelerometer Guide," [Online]. Available: https://www.phidgets.com/docs/Accelerometer_Guide . [Accessed: 2025].

[10] WatElectronics, "What is a Gyroscope Sensor: Working & Its Applications," 16 May 2020. [Online]. Available: https://www.watelectronics.com/what-is-a-gyroscope-sensor-working-its-applications/ . [Accessed: 4 October 2025].

[11] u-blox, "NEO-M8 Concurrent GNSS Modules Data Sheet," u-blox. [Online]. Available: https://content.u-blox.com/sites/default/files/NEO-M8_DataSheet_(UBX-13003366).pdf. [Accessed: 24 April 2026].

[12] XLSEMI, "XL4016 Datasheet — 180KHz 8A Step Down Voltage Regulator," XLSEMI. [Online]. Available: https://www.mikrocontroller.net/attachment/534859/XL4016_Step_Down_Buck_DC_DC_Converter.pdf. [Accessed: 10 May 2026].

[13] Wikipedia, "Google Colab," [Online]. Available: https://en.wikipedia.org/wiki/Google_Colab . [Accessed: 21 January 2026].

[14] JetBrains, "Kotlin Programming Language," [Online]. Available: https://kotlinlang.org/. [Accessed: 10 May 2026].

[15] Mozilla Developer Network, "XML Introduction," [Online]. Available: https://developer.mozilla.org/en-US/docs/Web/XML/Guides/XML_introduction . [Accessed: 10 May 2026].

[16] K. Rajeswari, D. N. Kumar, K. G. Kumar, K. D. Kumar, and K. K. Kumar, "IoT and Machine Learning," International Journal for Research Trends and Innovation, vol. 10, no. 4, p. 11, 2025.

[17] Ramsha Suhail, F. Ahmed, and H. Baig, "Automated Sensor Based Pothole Detection System for Preventing Unfortunate Causality," International Journal of Engineering Research & Technology, vol. 9, no. 7, p. 5, 2020.

[18] Atlassian, "Agile Epics: Definition, Examples, and Templates," Atlassian. [Online]. Available: https://www.atlassian.com/agile/project-management/epics. [Accessed: 17 November 2025].

[19] IEEE, "IEEE Standard for Software and System Test Documentation," IEEE, New York, 2008.

[20] Google, "Google Maps," Google LLC, February 2005. [Online]. Available: https://www.google.com/maps. [Accessed: 21 January 2026].

[21] L. Bass, P. Clements, and R. Kazman, Software Architecture in Practice, Boston: Addison-Wesley, 2013.

[22] M. Shaw and D. Garlan, Software Architecture: Perspectives on an Emerging Discipline, Englewood Cliffs: Prentice Hall, 1996.

[23] TDK InvenSense, "MPU-6050: 6-Axis MEMS MotionTracking Device Datasheet," TDK InvenSense, San Jose, 2013.

[24] Raspberry Pi Foundation, "Raspberry Pi 4 Tech Specs," Raspberry Pi Foundation, 2025. [Online]. Available: https://www.raspberrypi.com/products/raspberry-pi-4-model-b/specifications/ . [Accessed: 22 January 2026].

[25] M5Stack, "COM.GPS Module (NEO-M8N) with Antenna," M5Stack Technology Co., Ltd., Shenzhen, China. [Online]. Available: https://shop.m5stack.com/products/com-gps-module-neo-m8n-with-antenna . [Accessed: 1 May 2026].

[26] TalentCell, “12V Lithium-Ion Battery Specifications.” [Online]. Available: https://www.talentcell.com/lithium-ion-battery/12v/yb1203000-usb.html. [Accessed: 11-May-2026].

[27] Texas Instruments, “LM2596 SIMPLE SWITCHER® Power Converter.” [Online]. Available: https://www.ti.com/product/LM2596. [Accessed: 11-May-2026].

[28] Components101, “XL4015 DC-DC Step Down Module.” [Online]. Available: https://components101.com/modules/xl4015-dc-dc-step-down-module. [Accessed: 11-May-2026].

[29] S. Bruno, G. Loprencipe, P. Di Mascio, G. Cantisani, N. Fiore, C. Polidori, A. D'Andrea, and L. Moretti, "A Robotized Raspberry-Based System for Pothole 3D Reconstruction and Mapping," Sensors, vol. 23, no. 13, p. 5860, 2023.

[30] S. U. Ahmed, H. Ayaz, S. A. S. Rizvi, M. H. Hashmi, M. Humayon, and M. S. Salam, "Pothole Detection using Computer Vision and Raspberry Pi," Journal of Computing & Biomedical Informatics, vol. 7, no. 1, p. 5, 2024.

[31] Google, 2024, Google Play Console Help — Publish your app, Google LLC, accessed 25 May 2026, https://support.google.com/googleplay/android-developer/answer/9859348.